# Environmental monitoring

of the Industrial Area in Grundartangi

### Results of the Year 2025

2024.11.06

Report – Information Sheet

Document Key	Report Title

114100-SKY-01-V01	Environmental Monitoring of the Industrial Area in Grundartangi

Results of the Year 2025

Report Number / Page Count	Project Title

ISBN 978-9935-9815-5-4 / 101	Environmental Monitoring in Hvalfjörður 2025

Project Manager / Buyer's Representative	Client

Magnús Freyr Ólafsson	Elkem Ísland ehf., Norðurál Grundartangi ehf., Alur Álvinnsla ehf.

EFLA Project Manager	Author

Eva Yngvadóttir	Eva Yngvadóttir, Páll Höskuldsson, Stefán Þór Kristinsson and Snævarr Örn Georgsson.

Keywords	Summary

Environmental Monitoring, Industrial Area in Grundartangi, Alur álvinnsla ehf., Elkem Ísland ehf., Norðurál Grundartangi ehf.

Report Status

- Draft

- Draft for review

☒ Done

Distribution

☒ Open

- Distribution with permission of the client

- Confidential

This report summarizes the results of monitoring carried out on the environment around the Grundartangi industrial area. The environmental monitoring is carried out according to an environmental monitoring plan approved by the Icelandic Environment and Energy Agency and valid until 2028. The companies involved in the environmental monitoring are Elkem Ísland ehf, Norðurál Grundartangi ehf, and Alur álvinnsla ehf.

The purpose of monitoring is to assess the environmental impact of activities in the Grundartangi industrial area.

The following factors were monitored during the year 2025: air quality (atmosphere and precipitation), fresh water, seawater by coastal flow pits, vegetation (grass, leaves, and pine needles), and herbivores (sheep and horses).

Results for air quality, freshwater, grass and sea reveal that all reference limits set in regulations and operating permits were met. Icelandic reference values for precipitation, vegetation (leaves and needles) and herbivores are not defined.

Fluoride concentrations in vegetation were in all cases below the tolerance limits for vegetation.

Although the average concentration of fluoride in the jaw bones of sheep has increased in comparison with 1997, it is unchanged compared to 2007. There does not appear to be a clear relationship between the dental health of slaughtering sheep and the concentration of fluoride in the jawbones. There were no detectable effects of fluoride on the teeth or joints of live sheep and horses.

## Summary

Environmental monitoring around the industrial area at Grundartangi is carried out according to an environmental monitoring schedule that is in effect until 2028. The purpose of the monitoring is to assess the impact of activities in the industrial area on the environment.  This year, measurements were made of air quality, fresh water, the environment of sea overflow pits, vegetation and herbivores.

The companies involved in the environmental monitoring are Alur álvinnsla ehf., Elkem Ísland ehf., and Norðurál Grundartangi ehf. They are all located in a planned industrial area in Grundartangi, where other activities also take place. In 2025, Alur recycled about 5,000 tons of aluminum from aluminum dross. Elkem's annual production was 104,800 tons of 75% silicon metal and 19,928 tons of silica dust, and Norðurál produced about 260,000 tons of aluminum.

This report summarizes the results of environmental studies in 2025 and other measurements that have been made since the beginning of monitoring. The results are compared with the reference limits in operating permits and regulations when available. A comparison is also made between years and with baseline measurements made in 1997, before the Norðurál smelter began operating, and in some cases compared to 2007, when all the smelter's pots were in operation.

Provisions on dilution zones have been repealed in the operating permits of Norðurál and Elkem, and therefore the limits stated in the operating permits of the companies apply outside the boundaries of the industrial area according to Hvalfjörður municipality's master plan.

Results for air quality, freshwater, grass and sea reveal that all reference limits set in regulations and operating permits were met. Icelandic reference values for precipitation, vegetation (leaves and needles) and herbivores are not defined. Fluoride concentrations in vegetation were in all cases below the tolerance limits for vegetation. The average concentration of fluoride in the jawbones of lambs has increased compared to 1997 but decreased compared to 2007. In adult sheep the average concentration of fluoride has increased compared to 1997 but remained unchanged compared to 2007. There does not appear to be a clear relationship between the dental health of livestock and the concentration of fluoride in the jawbone. There were no detectable effects of fluoride on the teeth or joints of live sheep and horses.

### Air quality

In all cases, the measured concentrations of gaseous fluorine, total fluorine, sulfur dioxide, hydrogen sulfide, nitrogen oxides,  particulate matter and benzo(a)pyrene were below the environmental limits defined in regulations or operating permits.

There are no defined environmental limits for the concentration of dissolved substances and the acidity of precipitation.  The measured values were highly fluctuating, the concentrations of fluoride and sulfate were quite high, and the precipitation was rather acidic. The concentration of other substances was in line with measurements from previous years.

### Freshwater

Overall, the results of the year do not indicate major changes in the pollution level of river surface water as compared with the outcome in recent years. The situation has been similar since 2006.

The mean value of fluoride in all the monitored rivers increases between years but is still in line with the average in 2016-2024. While the concentration of fluoride in Urriðaá river was in line with the average of measurements from 2016-2024, the concentration in Kalmansá was higher than the average over the same period. The acidity and average concentration of fluoride and sulfate in all monitored rivers were within the limits defined in the  Icelandic Drinking Water Regulation (IDWR). The fluoride concentration in the rivers Laxá, Fossá, Berjadalsá, and Kúludalsá was in line with the measuring values of previous years and was about one-fifth of the fluoride concentration in rivers Kalmansá and Urriðaá. The measurement results do not indicate major changes in the pollution level of the monitored rivers.

### Seawater near overflow pits

The results from measurements in 2025 are similar to previous years and indicate that pollution is insignificant in the vicinity of the overflow pits. Samples showed that the fluoride concentration was below the concentration that is considered to have an impact on sensitive ecosystems and they are within the limits specified in Norðurál's operating license. Other substances that were found proved in most cases to be in similar concentrations to the reference samples.

### Vegetation

Grass

In all cases, the value of fluoride in grass was below the estimated exposure limits for grass and below the regulatory limits for fluoride in feed. Fluoride concentration in grass in 2025 increased slightly from 2024. While there was an increase in the average fluoride concentration in leaves both north and south of Hvalfjörður fjord compared to 1997, there was no change compared to 2007.

Leaves

The fluoride concentration in leaves that was measured in 2025 proved on average to be higher than the previous year and yet, in all cases, below the tolerance limits for deciduous trees. There was an increase in the average fluoride concentration in leaves north and south of Hvalfjörður fjord compared to 1997, but no change compared to 2007.

Conifers

The average fluoride concentration in one-year and two-year-old conifers was lower in 2025 compared to 2024 and in all cases below the tolerance limits of conifers. There was no significant change in the average fluoride concentration in one-year or two-year-old conifers both north and south of the fjord compared to 1997 and 2007.

### Herbivores (sheep and horses)

There are no defined reference limits in Icelandic regulations for the fluoride concentration in the jawbones of lambs or adult sheep.

Fluoride in lambs

There does not appear to be a clear relationship between the dental health of lambs and the concentration of fluoride in their jawbone. According to a Norwegian study, fluoride in deer was in all cases found to be below the limits where it may pose a risk for dental caries. The average fluoride concentration in 2025 in lambs from all monitoring areas was within the range that measurements have shown since 2008.

Whereas in 2025, there was an increase in the average fluoride concentration in leaves north and south of Hvalfjörður fjord compared to 1997, it was still lower than in 2007.

Fluoride in adult livestock

There does not appear to be a clear relationship between the dental health of adult sheep and the concentration of fluoride in their jawbone. The average concentration of fluoride in adult sheep from four farms was above the level considered to pose a risk of dental caries when it comes to deer. No animal turned out to have the concentration of fluoride considered to cause dental caries in deer.

The average fluoride concentration in 2025 was measured within the fluctuation that measurements have shown since 2008. In 2025, there was an increase in the average fluoride concentration in adult sheep north of Hvalfjörður compared to 1997, but no change compared to 2007. South of the fjord, there was a significant increase in the average fluoride concentration in adult sheep compared to 1997 and 2007.

Live sheep and horses

The results of a veterinarian’s examination of the teeth and foreleg joints of live herbivores (sheep and horses) indicated that there were no detectable effects on these caused by exposure to fluoride. The condition of the teeth and joints was within the limits that the veterinarian considered as normal.

The scope of environmental monitoring in 2025 covered about 120 sampling points, where around 420 samples were collected and analyzed. Studies for environmental monitoring were carried out by the Marine Research Institute's Chemical Analysis Department, Iceland Food and Biotech R&D (Matís), the Mosfellsbær Veterinary Clinic and other independent parties. An overview of the results of the environmental monitoring of the industrial area in Grundartangi for the year 2025 is provided in the table below. Provisions in Icelandic regulations or in the companies' operating permits are in place for over 30 of the parameters measured. For those parameters where Icelandic environmental limits have not been defined, the results were compared with background values and results from previous years.

| Notes |  |  |
| --- | --- | --- |
| No reference values in Icelandic regulations or operating permits | Provisions in Icelandic regulations and operating permits met | Provisions in Icelandic regulations or operating permits not met |

|  |  | Parameters | Provisions in Icelandic regulations and operating permits |
| --- | --- | --- | --- |
| Air quality | Atmosphere | Gaseous fluoride in the atmosphere outside the industrial site | Norðurál’s operating permit |
|  |  | Gaseous fluoride & fluoride in particulate matter outside the industrial site | Norðurál’s operating permit |
|  |  | Sulphur dioxide in atmosphere and dust | Annual average concentration - Vegetation protection limit cf. reg. No. 920/2016 |
|  |  |  | Daily mean value SO2- Health risk limits cf. reg. No. 920/2016 |
|  |  |  | Hourly mean value SO2 - Health risk limits cf. reg. No. 920/2016 |
|  |  | Hydrogen sulphide | Average annual concentration H2S - Health risk limits cf. reg. No. 514/2010 |
|  |  |  | Hourly mean value H2S - Health risk limits cf. reg. No. 514/2010 |
|  |  | Nitrogen dioxide | Average annual concentration NO2 - Health risk limits cf. reg. No. 920/2016 |
|  |  |  | Hourly mean value NO 2- Health risk limits cf. reg. No. 920/2016 |
|  |  | Nitrous oxide | Annual average concentration NOX - Vegetation protection limit cf. reg. No. 920/2016 |
|  |  | Particulate matter (PM10) | Average annual concentration - Health risk limits cf. reg. No. 920/2016 |
|  |  |  | Hourly mean value - Health risk limits cf. reg. No. 920/2016 |
|  |  | Particulate matter (PM2,5) | Average annual concentration - Health risk limits cf. reg. No. 920/2016. |
|  |  | PAH16 | Environmental limits for benzo(a)pyrene cf. reg. No. 410/2008 |
|  | Precipitation | Chloride | Maximum value not set in Icelandic regulations |
|  |  | Sodium |  |
|  |  | Calcium |  |
|  |  | Sulfate |  |
|  |  | Fluoride |  |
|  |  | pH |  |
| Freshwater |  | Conductivity | Maximum value not set in Icelandic regulations |
|  |  | pH | The Icelandic Drinking Water Regulation, No. 536/2001 |
|  |  | Fluoride |  |
|  |  | Chloride |  |
|  |  | Sulfate |  |

|  | Parameters | Provisions in Icelandic regulations and operating permits |
| --- | --- | --- |
| Sea near overflow pits | Aluminum | Norðurál’s operating permit |
|  | Fluoride |  |
|  | Arsenic | Icelandic Regulation No. 796/1999 (with subsequent amendments) regarding the prevention of water pollution |
|  | Lead |  |
|  | Copper |  |
|  | Chromium |  |
|  | Nickel |  |
|  | Zinc |  |
|  | Iron | The Icelandic Drinking Water Regulation, No. 536/2001 |
|  | Cyanides |  |
|  | Vanadium | Maximum value not set in Icelandic regulations |
|  | Phosphorus |  |
| Vegetation | Fluoride in grass used as fodder for sheep | Icelandic Regulation No. 1206/2021 (which enacts EU Directive 2002/32/EC into Icelandic law) on undesirable substances in animal feed |
|  | Fluoride in leaves | Maximum value not set in Icelandic regulations |
|  | Fluoride in pine needles |  |
| Herbivores | Fluoride in the jawbones of sheep | Maximum value not set in Icelandic regulations |
|  | Condition examination of the teeth and jaws of slaughtering sheep |  |
|  | Condition examination of the teeth of live sheep |  |
|  | Condition examination of the joints of live sheep |  |
|  | Condition examination of the teeth of live horses |  |
|  | Condition of the joints in live horses |  |

# Table of Contents

SUMMARY	5

DEFINITIONS	13

- INTRODUCTION	15

- LOCAL CONDITIONS	16

  - INDUSTRIAL AREA	16

  - Operations	17

  - Weather condition	17

- ENVIRONMENTAL MONITORING AND MEASUREMENT PARAMETERS	20

  - Monitoring sites and parameters	20

  - Statistics	21

- AIR QUALITY	22

  - Results	23

    - Fluoride in the atmosphere	23

    - Sulfur dioxide in the atmosphere over the year	26

    - Hydrogen sulfide in the atmosphere over the year	28

    - Nitrogen oxide in the atmosphere	30

    - Particulate matter in the atmosphere	31

    - Polyaromatic hydrocarbons (PAH) in the atmosphere	32

    - Dissolved substances and pH in precipitation	33

- FRESHWATER	36

  - Results	37

    - Freshwater conductivity	37

    - Freshwater pH	38

    - Fluoride in freshwater	39

    - Sulfate in freshwater	40

- MARINE MONITORING AT THE FLOW PITS	41

  - Results	42

    - Metals and Other Elements in Seawater Samples	42

    - Cyanide in seawater samples	43

- VEGETATION	45

  - Results	46

    - Fluoride in grass	46

    - Fluoride in foliage	48

    - Fluoride in pine needles	49

- HERBIVORES (SHEEP AND HORSES)	51

  - Results	53

    - Fluoride in lambs	54

    - Fluoride in adult sheep	55

    - Inspection of jaws and teeth in slaughtering sheep	56

    - Inspection of teeth and joints in live sheep and horses	57

- BIBLIOGRAPHY	58

APPENDIX A	OLDER ENVIRONMENTAL MONITORING REPORTS	60

APPENDIX B	AIR QUALITY	61

APPENDIX C	FRESHWATER	68

APPENDIX D	SEAWATER BY OVERFLOW PITS	73

APPENDIX E	VEGETATION	75

APPENDIX F	HERBIVORES (SHEEP AND HORSES)	86

## Definitions

Definitions:

Background values	Concentrations of substances that are considered to exhibit a    natural value.

BaP	Benzo(a)pyrene, a component of PAH16.

Vegetation protection limit	 Limits aimed at protecting vegetation against harmful effects, according to Regulation No. 920/2016.

Health risk limits	Limits set to ensure human health in the long term.1

pH		Acidity determined by the amount of hydrogen ions in an aqueous solution.

Particulate matter (PM10)	Particulate matter in the air that is 10 µm or less in diameter.

Particulate matter (PM2,5)	Particulate matter in the air that is 2,5 µm or less in diameter.

Gaseous fluoride (HF)	The fraction of fluoride measured as gaseous matter and free dust

Fluoride particulate (F)	The fraction of fluoride measured as bound dust

Total fluoride	Total gaseous and particulate fluoride

Environmental limits	Environmental limits are the maximum permissible level of contamination in a given receptor to prevent or mitigate harmful effects on human health and/or the environment. Environmental limits are set to protect the environment as a whole or specific components of it (such as health risk limits and vegetation protection limits for the protection of ecosystems).

Receptor	An area that receives pollution and dilutes or eliminates/neutralizes it.

Tolerance limits	The maximum concentration of a contaminant in the tissue of organisms that they are expected to tolerate without harm.

Regulations:

920/2016	On sulfur dioxide, nitrogen dioxide and nitrogen oxides, benzene, carbon dioxide, particulate matter and lead in the atmosphere, the concentration of ozone at the earth's surface and information for the general public.

514/2010	On the concentration of hydrogen sulphide in the atmosphere.

265/2010	Sets maximum levels for certain contaminants in foodstuffs.

410/2008	On atmospheric arsenic, cadmium, mercury, nickel and polycyclic aromatic hydrocarbons (PAHs).

536/2001	On drinking water.

1206/2021	On undesirable substances in animal feed (which enacts EU Directive 2002/32/EC into Icelandic law)

796/1999	On the prevention of water pollution

1 Health risk limits in Regulation 920/2016 are different from the pollution limits stipulated in Regulation 390/2009 on pollution limits and measures to reduce pollution in the workplace, which is defined as the highest permissible average pollution (time-weighted average) in the atmosphere of employees [mean value or ceiling value].

# Introduction

The companies Elkem Ísland ehf., Norðurál Grundartangi ehf. and Alur álvinnsla ehf. operate in a designated industrial area in Grundartangi, Western Iceland. The operations of these companies are subject to operating permits that stipulate regular monitoring of the environment in the vicinity of the industrial area.

The monitoring for the year 2025 was conducted according to an environmental monitoring plan that is valid until 2028 and is carried out according to the companies' operating permits and is confirmed by the Icelandic Environment and Energy Agency [1]. The objective of environmental monitoring is to assess the impact that the companies' operations have on the environment.

This report contains a summary of the results of monitoring studies in the vicinity of the industrial site, which were carried out for the most part in 2025. The results are compared with reference values, where applicable, as well as the results of previous years. The variability of measurements for freshwater, vegetation and sheep is also assessed using a statistical approach. All primary data on which the summary is based are published on the website of the Icelandic Environment and Energy Agency.

The report is divided into eight sections. Section 1 contains a short introduction describing the purpose of environmental monitoring. Section 2 entails a description of the local conditions and climate at the industrial site. Section 3 provides an overview of the scope and measurement parameters of the environmental monitoring Sections 4–8 summarize the measurements and results of monitoring for air quality, freshwater, seawater at the low pits, vegetation and herbivores. Finally, the appendices of the report provide detailed information and further information related to the measurements and results. Appendix A contains a list of previously published annual environmental monitoring reports. Appendix B contains additional information on atmospheric and precipitation monitoring. Results of freshwater measurements and statistical data can be found in Appendix C. Appendix D contains additional information on marine monitoring at flow pits.  Appendix E provides information on vegetation and statistical data, and Appendix F contains additional information and statistical analysis of herbivore examination results.

# Local conditions

### Industrial area

The designated industrial area at Grundartangi is located on the northern side of the fjord Hvalfjörður, as Figure 2.1. shows.

FIGURE 2.1	Location of the industrial area at Grundartangi in Hvalfjörður County (in dark grey).

All provisions on dilution zones were repealed in 2020 with amendments to Regulation No. 787/1999 on air quality and Regulation No. 514/2010 on the concentration of hydrogen sulfide in the atmosphere. The Icelandic Environment and Energy Agency removed the dilution zone provisions from Elkem and Norðurál's operating licenses on 19 June 2025.

Norðurál's operating license includes limits for gaseous fluoride and total fluoride, which apply outside the boundaries of the industrial area defined in the Hvalfjörður Land use Plan 2020-2032. [2].

In Hvalfjörður County’s Land Use Plan, valid for the period 2020–2032, former dilution zones are defined as “caution zones”, which include restrictions on land use due to risks to public health and safety, as well as polluting industrial activities [2].

### Operations

Elkem's silicon metal plant has been operating in Grundartangi since 1979. The plant's annual production capacity is 120,000 tons of silicon metal. Elkem in Grundartangi produces, among other things, a specialized silicon metal alloy that is important for the global electric vehicle market and the production of energy-efficient household appliances. In 2025, Elkem produced 104,800 tons of 75% silicon metal and 17,928 tons of silica powder. The fume treatment plant was in operation for 99.94% of the combined operating time of all the company’s furnaces. Elkem’s operating permit allows for the annual production of 190,000 tons of silicon metal and 45,000 tons of silica dust. During the production of silicon metal, emissions into the environment are mainly airborne, which can mostly be attributed to silicon metal dust, sulfur dioxide (SO2) and carbon dioxide (CO2).

In 2025, over 700 people worked at Norðurál in Grundartangi, in a variety of jobs, producing 260 thousand tons of aluminum. The product is pure aluminum and aluminum alloys. Norðurál emphasizes that the company's operations are in harmony with the environment and constantly endeavors to minimize environmental impact. Stability in operations and the use of renewable energy sources ensure that the carbon footprint of Icelandic aluminum is among the lowest in the world. In 2020, Norðurál received ASI certification (Aluminium Stewardship Initiative) for responsible and environmentally friendly production and was recertified according to the updated ASI Performance Standard in 2023. The certification confirms that the company's operations meet strict requirements for social and environmental responsibility, and good practices in aluminum production. In 2022, Norðurál received The Icelandic Business Environment Award, among other things for the production of aluminum with a low carbon footprint that has been traced from the beginning to the end of the production cycle in a life cycle analysis. In 2023, Norðurál received Fjörusteinninn, Faxaflóahafnir’s (Associated Icelandic Ports) environmental recognition, which is awarded to companies that demonstrate progress in environmental matters and are exemplary in terms of a clean environment. The Norðurál smelter in Grundartangi was commissioned in 1998 and has a permit to produce 350,000 tons of aluminium per year.

Alur álvinnsla ehf. is an aluminum recycling company in the Grundartangi Industrial Area. The company's core activity is the recycling of aluminium dross that is generated during the primary production of aluminum. Alur produces about 5,000 tons of aluminium annually, which is fed back into the production cycle of the smelters.

### Climate

Meteorological measurements are taken at automatic weather monitoring stations at Kríuvarða, Gröf II and at Grundartangi Harbour. Figure 3.1. shows the location of the air quality monitoring stations. The Gröf II measuring station is located about 3 km southwest of Grundartangi, close to the farm Gröf II at an altitude of about 40 m above sea level. The measuring station at Kríuvarða is located about 700 meters north of the eastern end of the aluminum smelter at an altitude of about 15 m above sea level; the measuring station at Grundartangi Harbour is located at an altitude of about 25 m above sea level. Overall, the average temperature for the year 2025 was similar to previous years. The same applies to the average wind speed for the year.

Weather conditions were quite favorable in the winter of 2024-2025, with relatively calm weather. The temperature in the spring (April and May) was the highest since records began. Precipitation was below the average for the years 1991-2020; the spring was sunny and vegetation was thriving well. The summer was relatively warm. In Reykjavík, precipitation was close to average. The autumn and winter months were warm except for November. Overall, 2025 was the warmest year in Iceland since records began.

Spring was unusually mild. The annual precipitation was below the average for the last ten years, and it was a year of light snow. The year was generally mild with only a few severe storms.

Table 2.1 shows the average temperature for the year and the monitoring period, from April to October, and the average wind speed at all measuring stations [3]. Wind speed was highest at Grundartangi Harbour, and the coldest temperature at Gröf II.

TABLE 2.1    Average temperature and average wind speed at weather stations in Hvalfjörður in 2025 [3].

| Weather station | January - December |  | April - October |  |
| --- | --- | --- | --- | --- |
|  | Mean temperature [°C] | Average wind speed [m/s] | Mean temperature [°C] | Average wind speed [m/s] |
| Grundartangi Harbor | 6.2 | 5.1 | 8.4 | 4.7 |
| Gröf II | 3.8 | 4.6 | 6.5 | 4.3 |
| Kríuvarða | 7.1 | 5.9 | 9.2 | 5.4 |

Figure 2.2 shows wind roses for the entire year 2025 and for the period April to October at all weather stations. While the most common wind directions at Gröf were north-northeast, they were easterly at Kríuvarða and in Grundartangi. The wind rose at Gröf II follows the mountain slope of Akrafjall, which obviously shapes the wind rose in a decisive way. There, easterly winds turn to northeasterly winds, and southerly winds turn to southwesterly winds due to the mountain's location. The wind roses for the monitoring period (April - October) and the wind roses for the entire year are very similar. Whereas at Gröf II, northeasterly winds are most common, easterly winds dominate in Grundartangi Harbour and at Kríuvarða [3].

FIGURE 2.2	Wind roses from Kríuvarða, Gröf II and Grundartangi Harbour for the period April - October 2025 and the entire year 2025.

# Environmental monitoring implementation and measurement parameters

Environmental monitoring for the year 2025 was carried out in accordance with the environmental monitoring plan that is valid until 2028 and can be found on the Icelandic Environment and Energy Agency’s homepage [1]. Table 3.1 provides an overview of the parameters, monitoring points, research players and research periods.

### Monitoring sites and parameters

In 2025, air quality, freshwater, seawater at flow pits, vegetation and herbivores (sheep and horses) were monitored. Figure 3.1. shows an overview of the monitoring sites. About 420 samples from about 120 sampling points were examined. Studies for environmental monitoring were carried out by the Marine Research Institute, Iceland Food and Biotech R&D (Matís), the Mosfellsbær Veterinary Clinic and other independent parties. Around 75 parameters were analyzed in the samples, see Table 3.1.

FIGURE 3.1	Overview of monitoring points for 2025 for air quality, freshwater, seawater at flow pits, vegetation and herbivores.

TABLE 3.1    Overview of environmental monitoring in Hvalfjörður in 2025.

|  | Parameters | Location of monitoring points | Monitoring period | Research player |
| --- | --- | --- | --- | --- |
| Air quality | Atmosphere: Fluoride in air and dust, sulfur dioxide in air and dust, particulate matter, PAHs in particulate matter, nitrous oxide and hydrogen sulfide Precipitation: Chloride, nitrate, sulfate, fluoride, pH, sodium, and calcium | North of Hvalfjörður: Kríuvarða, Gröf II, Melahverfi | Kríuvarða, Gröf II and Melahverfi: throughout the year | Sampling and chemical analysis: The Marine Research Institute and Matís (the Centre for Measurement, R&D for food production in Iceland)* |
|  |  | South of Hvalfjörður: Hálsnes | Every third year: Not monitored in 2025 |  |
| Freshwater | PH, conductivity, fluoride, chlorine, sulfate, sodium and calcium | Streams without glacial water Berjadalsá, Fossá, Laxá, Kúludalsá Streams with sources in surface water: Kalmansá, Urriðaá | 13 April - 12 September 2025 | Sampling and chemical analysis: The Marine Research Institute and Matís |
| Sea near flow pits | Metals: arsenic, aluminum, lead, copper, chromium, nickel, iron, zinc, and vanadium Cyanide, fluoride and phosphorus | Water samples taken at 10 locations near flow pits plus two reference sites | 26 May, 15 July, and 28 July 2025 | Sampling and chemical analysis: The Marine Research Institute and Matís |
| Vegetation | Fluoride (in plant web and from the surface) in grass, leaves (birch, spruce) and conifers (spruce, mountain pine, lodgepole pine) | North of Hvalfjörður: Stekkjarás, Fannahlíð, Fellsaxlarkot, Ferstikla, Hólabrú, Gröf II by the farmhouse and road, Hlíð, Kalastaðakot South of Hvalfjörður: Félagsgarður, Fossbrekka, Háls in Kjós, Reynivellir Reference samples: Skorradalur | Grass: 5 June, 7 July, 1 September 2025 Leaves: 5 June, 1 September 2025 Pine needles: 19 November 2025 | Sampling and chemical analysis: The Marine Research Institute, Matís and Land and Forest Iceland |
| Herbivores (sheep and horses) | Fluoride in the jaws of slaughtering sheep and the condition of teeth and jaws Condition of teeth and joints in live sheep Condition of teeth and joints in live horses | Slaughtering sheep: North of Hvalfjörður: Skorholt, Eystri Leirárgarðar, Hóll, Skipanes, Vogatunga, Eystra Miðfell, Hrafnabjörg, Vestri Reynir, Innri Hólmur South of Hvalfjörður: Kiðafell, Miðdalur, Fell Reference samples: Bjarnarhöfn in Snæfellsnes peninsula and Skjaldfönn in Ísafjarðardjúp. Live sheep: North of Hvalfjörður: Vogatunga, Eystra Miðfell, Hrafnabjörg South of Hvalfjörður: Kiðafell, Miðdalur, Fell Live horses: North of Hvalfjörður: Litla Fellsöxl, Kalastaðakot, Ytri Hólmur South of Hvalfjörður: Miðdalur, Meðalfell | Slaughtering sheep: Fall slaughtering 2025 Live sheep and horses: January and February 2026 | Chemical analysis: Matís (Centre for Measurement, R&D for food production in Iceland) Examination of sheep: The Veterinarian in Mosfellsbær Examination of live herbivores: The Veterinarian in Mosfellsbær |

### Statistics

Since 1999, annual environmental monitoring of freshwater, vegetation and herbivores has been carried out. Baseline research was conducted in the area in 1997–1998. An assessment of the variability of the measurements is made on the basis of statistical analysis of the results compared to 1997, before the Norðurál smelter began operation, and in some cases compared to 2007, when all the smelter's pots were in operation. A statistical methodology is used to calculate a 95% safety margin for the averages of measurements. Appendices C, E and F show the results of the statistical analyses.

# Air quality

Three air quality monitoring stations were in operation in 2025: at Kríuvarða, Gröf II and in Melahverfi (Figure 4.1). These monitoring stations, among other things, performed automatic measurements throughout the year for different parameters (Table 4.1). Continuous, automatic measurements are carried out at Kríuvarða, northeast of the industrial site at Grundartangi, and at Gröf, southwest of Grundartangi. Measurements of sulfur dioxide and hydrogen sulfide are conducted in Melahverfi. The air quality monitoring station at Hálsnes is operated every three years, according to the monitoring plan; the last one was in 2024, and the next will be in 2027.

FIGURE 4.1	Location of air quality monitoring stations.

Sampling and measurements were performed by Matís [3]. The results of chemical analyses were compared with the reference values stated in the operating permit.

TABLE 4.1	Measurement parameters at air quality monitoring stations that were in operation in Hvalfjörður in 2025. The results of measurements of SO2, H2S, NO2 and particulate matter (PM10, PM2,5), are published on www.loftgæði.is.

| Air quality measuring station | Automatic measurements all year round | Samples collected in filters throughout the year | Precipitation |
| --- | --- | --- | --- |
| Kríuvarða | Particulate matter (PM10, PM2,5), SO2, H2S, NO, NOx | HF and F, S and PAH16 materials in particulate matter | NO3, Na, Ca, Cl, SO4, F, pH, conductivity |
| Gröf II |  | HF and F and S in PM |  |
| Melahverfi | SO2, H2S |  |  |
| Hálsnes (April-October) |  |  |  |

There were a number of failures in measuring equipment during the year, including [3]:

- Failure of automatic equipment that affected PM10 measurements at Kríuvarða. Consequently, PM10 measurement data is only available for the period January to April.  Data for PM2.5  is available for the period January to November.

- Automatic measurements of PM₁₀ and PM₂.₅ at Gröf were not carried out during the period April to June due to equipment failure.

- Automatic measurements of SO₂ and H₂ in Melahverfi were carried out from September to December due to technical difficulties earlier in the year.

- Equipment failures affected NOx measurements at Gröf, so data is only available for the period January to May and June to August.

- Due to technical difficulties, PAH measurements were not conducted at Kríuvarða in the period October to December.

- Fluoride deposition load at Kríuvarða and Gröf II is not available for the period March to May and for August.

- The deposition load for sulfate, sodium and chloride at Kríuvarða and Gröf II is not available for the period July to December.

- pH measurements of the acidity of precipitation are not available for April, May and July.

These measurement gaps must be considered when the measurement results for the year are reviewed and compared with previous years. The environmental monitoring operators in Hvalfjörður informed the Icelandic Environment and Energy Agency of the failures as soon as they were known. The operators have invested in new equipment to replace the failed one and the installation took place at Kríuvarða in late autumn 2025.

### Results

A summary of the results of air quality measurements (atmosphere and precipitation) together with defined environmental limits can be found in sections 4.1.1 – 4.1.7. Appendix B shows more detailed results of air quality measurements. Tables B.1 - B.3 provide an overview of the regulations that stipulate the relevant limit values for each parameter, as well as the highest and lowest measured values. The graphs and charts relevant to air quality show the results for the last 10 years. They refer to annual environmental monitoring reports for the years 1998 to 2015, which can be accessed on the Icelandic Environment and Energy Agency’s website.

    - Fluoride in the atmosphere

Norðurál’s operating permit defines the environmental limit for gaseous fluorine (HF) as 0.30 μg/m³ and for total fluorine (F) as 0.40 μg/m³, based on the period 1 April to 30 September. These environmental limits apply outside the boundaries of the industrial area according to the Hvalfjörður Municipality Master Plan [2]. On 19 June 2025, the Icelandic Environment and Energy Agency repealed the dilution zone provisions from the operating permits of Elkem and Norðurál, which means that the environmental limits in Norðurál's operating permit do not apply to the monitoring station at Kríuvarða, which is located within the industrial area.

Although fluoride measurements are conducted throughout the year, this section of the report discusses fluoride measurements from April to September in accordance with the defined environmental limits in the operating permit. Appendix B shows the average concentration of fluoride (HF) for each month throughout 2025.  There are no environmental limits for fluoride that apply throughout the year.

Samples for fluoride measurements were collected in filters at Kríuvarða and Gröf II according to the monitoring plan. The concentration of gaseous fluorine at Kríuvarða was measured at an average of 0.38 µg HF/m3 from April to the end of September. The environmental limits (0.3 µg HF/m3 in the period April-September) defined in Norðurál's current operating permit do not apply to Kríuvarða, cf. Figure 4.2. This is the highest concentration of gaseous fluorine measured since 2021. Whereas the highest average monthly concentration of gaseous fluorine was in July at 0.56 μg/m³, the highest daily value was in May at 3.38 μg/m³. The average concentration of HF at Gröf II was 0.03 μg HF/m3, which is equal to the average of the last 10 years and well within environmental limits (Figure 4.2).

1.0

µg HF/m³

0.8

0.6

0.4

0.2

0.0

HF in the atmosphere

Kríuvarða

Exposure limit according to operating permit

1997	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

1.0

µg HF/m³

0.8

0.6

0.4

0.2

0.0

Gröf II

Hálsnes

Exposure limit according to operating permit

1997	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

FIGURE 4.2	Average concentration of gaseous fluoride (HF) in the atmosphere in the period April – September at Kríuvarða in the top image and Gröf II and Hálsnes in the bottom image. The operating permit limits do not apply to Kríuvarða as the provisions in Norðurál's operating permit apply outside the industrial area. Kríuvarða is located within an industrial area according to the Hvalfjörður Municipality's master plan 2020-2032.

Figure 4.3 shows the concentration of total fluoride (F) over the reference period, i.e. the combined concentration of gaseous fluoride (HF) in the atmosphere and fluoride bound in dust. The average concentration of total fluoride at Kríuvarða was 0.43 µg F/m3 in the period 1 April - 30 September. The environmental limits (0.4 µg HF/m3 over the period April-September) defined in Norðurál's current operating permit do not apply to Kríuvarða. At Gröf II, the average concentration of total fluoride was 0.04 µg F/m3, which is well within environmental limits (Figure 4.3) and equal to the average for the last 10 years.

Total fluoride in the atmosphere

1.0

0.8

µg Total/m³

0.6

0.4

0.2

0.0

Kríuvarða

Exposure limit according to operating permit

1997	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

1.0

0.8

µg Total/m³

0.6

0.4

0.2

0.0

Gröf II

Hálsnes

Exposure limit according to operating permit

1997	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

FIGURE 4.3	Average concentration of gaseous fluoride (HF) in the period April – September at Kríuvarða in the top image and Gröf II and Hálsnes in the bottom image. The operating permit limits do not apply to Kríuvarða as the provisions in Norðurál's operating permit apply outside the industrial area. Kríuvarða is located within the industrial area according to the Hvalfjörður municipality's master plan 2020-2032.

Wind directions have different effects on fluoride measurements at the stations. Fluoride concentrations were measured higher at Kríuvarða than at Gröf II, as before, which is due to the proximity and position to the industrial area. At Kríuvarða, high fluoride concentrations are most likely to occur in southwest and south-southwesterly winds. At Gröf, higher concentrations are measured in northeasterly winds. The highest total fluoride was measured at 3.38 µg F/m3, on May 9 at Kríuvarða in southwesterly winds. Figure 4.4 shows the distribution of measured values by wind direction for all measurement days  [3].

FIGURE 4.4	Total fluoride (F) concentrations by wind direction in 2025 at Gröf II and Kríuvarða. The graphs are not on the same scale.

    - Sulfur dioxide in the atmosphere over the year

Sulfur dioxide (SO2) was measured by automatic continuous measurements throughout the year at the measuring stations by Kríuvarða and Gröf II. In Melahverfi, a measuring device failed in mid-July 2024 and was out of service until September 2025 (Figure 4.5 in this report and Figure B.2 in Appendix B). The reference limits are based on the average value for each year.

SO2 in the atmosphere

25

20

µg SO₂/m³

Vegetation protection limits

15

Kríuvarða	Gröf II	Melahverfi	Hálsnes

10

5

0

1997

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

FIGURE 4.5	Average SO2 concentrations at Kríuvarða, Gröf II, Melahverfi and Hálsnes.

SO2 concentrations were well below the vegetation protection limit (20 μg SO₂/m³)  at all measuring stations. The average concentration at Kríuvarða and Melahverfi was lower than in recent years but at Gröf, it was like the previous years. For the first time since the beginning of measurements, the average concentration at Gröf was higher than at Kríuvarða (Figure 4.5).

The hourly health protection limit is 350 μg SO2/m3 and it is permissible to exceed that limit 24 times during the year. In total, there were 17 hourly measurements at Gröf that exceeded the limit in 2025, 16 of them on July 20-21, when a volcanic eruption was taking place in Reykjanes. During that period, the highest hourly values are also at Kríuvarða, with the highest being 160 µg SO2/m³ in the early morning of July 21st. At Melahverfi, the highest concentration was 17.1 µg SO₂/m³ on December 22. Therefore, health protection limits were not exceeded at those measurement points.

The health protection limit for the 24-hour average concentration of SO2 is 125 µg SO2/m3, but this limit can be exceeded three times in one year. Figure 4.6 shows the frequency of measurements for different concentrations of SO2 at Kríuvarða, Gröf II and Melahverfi in 2025. Measurements are recorded every day of the year, but sometimes some days are missed due to tending and maintenance, which is reflected in the fact that the total number of measurements is not the same as the number of days in the year. In 2025, two days were missing at Kríuvarða, eight days at Gröf II, and 253 days in Melahverfi due to a failure. The daily average SO2 concentration exceeded the health risk limit twice at Gröf, on 20 and 21 July 2025, in southwesterly winds.

400

350

300

250

Number of days

200

150

100

50

0

Kríuvarða	Gröf II	Melahverfi

349 347

Health risk limits

125 μg SO2/m3

112

10  8

3

1

2

<25	25 - 50	50 - 75	75 - 100	100 - 125

µg SO₂/m³

>125

FIGURE 4.6	Number of measurements for different daily average SO2  concentrations at Kríuvarða, Gröf II and Melahverfi in 2025.

The main source of SO2 at Kríuvarða and Gröf II is the industrial area at Grundartangi, where SO2 reaches Kríuvarða in southwesterly winds and Gröf II in northerly winds (Figure 4.8). This year, it is likely that the volcanic eruption at Stóra-Skógarfell in Reykjanes peninsula in Southern Iceland may have affected the sulfur dioxide measurements over the year. In comparison, the sulfur dioxide concentration in this area is somewhat higher than that measured in the capital area, where averages in the range of <1-2 µg SO2/m³ are common [3].

    - Hydrogen sulfide in the atmosphere over the year

Automatic measurements were made of the concentration of hydrogen sulfide (H2S) in the atmosphere at Kríuvarða, Gröf II and Melahverfi throughout 2025. The average concentration of airborne H2S was measured below all defined environmental limits at all measuring points (Figure 4.7), i.e. health risk limits, 24-hour health risk limits and mandatory warning limits for the highest hourly average concentration, see tables B.1, B.2 and B.3 in Appendix B.

H2S in the atmosphere

5

Health risk limits

4

Kríuvarða	Gröf II	Melahverfi

µg H₂S/m³

3

2

1

0

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

FIGURE 4.7	Average concentration of H2S at Kríuvarða, Gröf II and Melahverfi.

The source of H2S at Kríuvarða and Gröf II is the geothermal areas near the capital area, southeast of the industrial site. The results are similar to previous years at all three measuring points. It should be noted that wind directions in Melahverfi are not measured at the measuring station itself, but rather in a weather station at Grundartangi Harbour (Figure 4.8).

Kríuvarða

Gröf II

Melahverfi

FIGURE 4.8	SO2 and H2S concentrations in measured wind directions in 2025 at Kríuvarða, Gröf II and Melahverfi. The graphs are not on the same scale.

    - Nitrous oxide in the atmosphere

Automatic measurements were made of the concentration of nitrous oxide (NOX) og and nitrous monoxide (NO) in the atmosphere at Kríuvarða throughout 2025, but due to a failure in the measuring equipment at Gröf II, measured values are only available for the period January to May and June to August. The difference between these substances is calculated as nitrous dioxide (NO2).

At both measuring stations, the average concentration of NO₂ was below the health risk limit (40 μg NO₂/m³) and the vegetation protection limit (30 μg NOx/m³), similar to that measured in 2024 (Figures 4.9 and 4.10)

40

Health risk limits

µg NO₂/m³

30

NO2 in the atmosphere

Kríuvarða	Gröf II

20

10

0

1997	2016	2017

2018

2019

2020	2021	2022

2023

2024

2025

FIGURE 4.9	Average concentration of NO2 at Kríuvarða and Gröf II.

NOx in the atmosphere

30

Vegetation protection limit

µg NOx/m³

20

Kríuvarða	Gröf II

10

0

1997

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

FIGURE 4.10    Average concentration of NOx at Kríuvarða and Gröf II.

The NO2 results were below the average of the past 10 years at both measuring points. The average concentration of NOx at Kríuvarða was higher than the previous year and higher than the average of the last 10 years. At Gröf, NOx decreased between years and the average concentration was below the average of the last 10 years. The main sources of nitrous oxides are most likely car traffic and activities in the industrial area.  Gröf II is close to traffic on Highway 1 and therefore higher concentrations can be expected there than at Kríuvarða. The diurnal average concentration of NO2 never exceeded the health protection limits at the monitoring stations, and the same applies to the hourly average concentration, see Tables B.1 and B.2 in Appendix B.

    - Particulate matter in the atmosphere

Automatic measurements of the concentration of particulate matter (PM10 and PM2.5) were carried out at Kríuvarða and Gröf II in 2025. The measuring device for PM10 at Kríuvarða was out of order from 10 April to the end of the year, and there were no measurements at Gröf II between 2 April and 24 June. These failures affect the measurements of particulate matter in the atmosphere. The average concentration of PM10 and PM2.5 turned out to be below the health risk limits throughout the year. PM10 concentrations were in line with the average values of the past 10 years at both Kríuvarða and Gröf II, but PM2.5 was above the average at Kríuvarða in 2025 (Figures 4.11 and 4.12).

Particulate matter (PM10) in the atmosphere

40

Health risk limits

µg PM10/m³

30

20

10

0

Kríuvarða	Gröf II	Hálsnes

1997	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

FIGURE 4.11	Average concentrations of particulate matter (PM10) in the atmosphere at Kríuvarða (from January – 10 April 2025), Gröf II and Hálsnes (not measured after 2018).).

20

Health risk limits

µg PM2,5/m³

15

Particulate matter (PM2.5) in the atmosphere

Kríuvarða	Gröf II

10

5

0

1997

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

FIGURE 4.12	Average concentrations of particulate matter (PM2.5) in the atmosphere at Kríuvarða and Gröf II. No measurements were received from Gröf II in 2024 due to a failure of the measuring instrument.

At the monitoring stations, the concentration of particulate matter PM10 never exceeded the 24-hour health risk limit during the year, see tables B.1 and B.2 in Appendix B.

At Kríuvarða, the concentration of particulate matter PM2,5 never exceeded the 24-hour health risk limit during the year (50 μg/m³), but it happened twice at Gröf, on 20-21 July when a volcanic eruption was taking place on Reykjanes peninsula, see tables B.1 and B.2 in Appendix B.

Figure 4.13 shows dust roses from Kríuvarða and Gröf II for the year 2025. At both stations, the concentration of particulate matter was highest in westerly and southwesterly winds. The sources of PM2.5 og PM10 are not necessarily the same, as the source of PM2.5 is related to combustion processes in vehicles or industry and can be far away, while the sources of PM10 are more in the local environment, e.g. from road or soil dust. Therefore, the likely sources of particulate matter at Kríuvarða and Gröf II are vehicle traffic near the industrial area and the capital area. Other sources are gravel quarries at Stóra-Fellsöxl and Hólabrú east of Hvalfjörður Tunnel, as well as construction work at the industrial area (Leandro Pellegrini, 2026).

Kríuvarða	Gröf

3

µg PM2.5/m

3

µg PM10/m

FIGURE 4.4	Total particulate matter concentrations by wind direction in 2025 at Gröf II and Kríuvarða. Note that the wind roses are not on the same scale.

    - Polycyclic aromatic hydrocarbons (PAH) in the atmosphere

Measurements were made on the concentration of polycyclic aromatic hydrocarbons (PAH16) in particulate matter samples (PM10) collected on filters at Kríuvarða in early 2025. Due to technical difficulties, samples were not collected from October to December. There are no defined limits for the total concentration of PAH16 in the atmosphere. The concentration of PAH16 in August and September is higher than all other measured values since the beginning of measurements at Kríuvarða and the average concentration is almost three times higher than the average of the previous ten years (Figure 4.14).

Environmental limits have been defined for benzo(a)pyrene (1 ng BaP/m3), which is one of the seven PAH16 substances that are recommended to be measured. BaP was measured higher than in recent years, or 0.0018 ng/m³, but still far below environmental limits.

PAH substances in the atmosphere

0.15

ng PAH/m³

0.10

Kríuvarða

0.05

0.00

1997	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

FIGURE 4.14	Concentration of PAH16 substances in particulate matter at Kríuvarða.

    - Dissolved substances and pH in precipitation

The concentration of sulphur, fluoride and pH in precipitation samples collected throughout the year at Gröf II and Kríuvarða, is measured. Over the years, these measured values have been fluctuating, as the sampling equipment is very simple and flies and pollen often get mixed up with the samples. The substances sodium, chlorine, calcium and nitrate [3] are also measured in the precipitation samples (see Appendix B.4). Results are calculated as the deposition load of pollutants per square meter per month at the location where the sample is taken, but pH is measured directly in precipitation samples. Results may reflect to some extent the amount of precipitation, as deposition load is higher in heavier precipitation.

In 2025, the heaviest rainfall occurred during southwesterly winds at Gröf II and southerly winds at Kríuvarða (Figure 4.15). Precipitation was quite uneven throughout the year and was not always the same at these two stations, although they are not far apart. The relative positions of the monitoring stations and the industrial area suggest that precipitation during easterly winds is more likely to result in elevated pollution levels at Gröf II, whereas precipitation during southwesterly winds is more likely to lead to increased pollution at Kríuvarða.

Kríuvarða	Gröf II

Precipitation mm/h Apr-Oct Precipitation mm/h Jan-Dec

FIGURE 4.15	Precipitation directions at Kríuvarða and Gröf II in the period April - October and the entire year 2025. The figures are not on the same scale.

Fluoride

No measurements of fluoride deposition load were recorded at either monitoring station during March–May or in August. The average fluoride deposition load in precipitation during April–October increased at both monitoring stations compared with the previous year and exceeded the average of recent years. The values measured fluctuated between years, as shown in Figure 4.16. The average value measured at Kríuvarða is similar to that observed in 2021, which represents the highest value within the reference period. No environmental limits have been defined for fluoride levels in precipitation.

Kríuvarða  Gröf II  Hálsnes

100

80

mg F/m²

60

40

20

0

1997

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

IMAGE 4.16	Deposition loads of fluoride in precipitation measured at Kríuvarða and Gröf II over the monitoring period (April–October).

Sulfate

There are no numerical observed values for sulfate deposition load after the month of July at both monitoring sites. The average sulfate deposition load in precipitation during the April–October monitoring period increased at both Kríuvarða and Gröf II compared with previous years and reached the highest average levels recorded in the past 10 years (Figure 4.17). The highest average monthly values were recorded at both sites in May and July. No environmental limits have been defined for sulfate in precipitation.

Sulfate in precipitation

Kríuvarða	Gröf II	Hálsnes

300

2-	2

250

mg SO4 –S/m

200

150

100

50

0

1997	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

IMAGE 4.17	Sulfate deposition load in precipitation at Kríuvarða, Gröf II, and Hálsnes during the monitoring period (April–October).

Acidity

No pH measurements are available for April, May, or July, all of which fall within the monitoring period. During the 2025 monitoring period, the pH measured at Gröf II was similar to the average for the preceding 10 years. In contrast, pH values at Kríuvarða were consistently lower than the 10-year average and showed little interannual variation, suggesting more acidic precipitation (Figure 4.18). No environmental limits have been defined for the acidity of precipitation. The Icelandic Meteorological Office monitors the acidity of unpolluted precipitation and monthly samples show a pH of 5.6. However, daily measurements of the same precipitation show an average of mild acidification (pH 5.4) [4].

14

12	Kríuvarða Gröf II Hálsnes 10

pH

8

6

4

2

0

1997	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

FIGURE 4.18	Average acidity of precipitation at Kríuvarða and Gröf II during the monitoring period (April – October).

# Freshwater

In 2025, six rivers were monitored for pH, conductivity, and the concentrations of fluoride, chloride, sulfate, sodium, and calcium. Five of these rivers—Laxá in Leirársveit (104-497-R), Urriðaá (104-138-R), Kalmansá (104-185-R), Berjadalsá above Akranes (104-179-R), and Kúludalsá (104-135-R)—are located north of Hvalfjörður. South of the fjord, samples were collected from Fossá below Eyrarfjall (Figure 5.1). The numbers following the river names refer to the corresponding water body identification numbers; Fossá has not been assigned a water body number. The sampling period extended from 13 April to 12 September 2025. Urriðaá and Kalmansá were sampled monthly, yielding a total of six samples from each river. Laxá and Fossá were sampled twice during the monitoring period, while three samples were collected from both Berjadalsá and Kúludalsá. Sampling and analyses were conducted by ha the Marine Research Institute and Matís [5]. Background data are available from 1997 for all monitored parameters in all rivers except Fossá, where measurements commenced in 2000, and Kúludalsá, where monitoring began in 2016.

Precipitation during the monitoring period was close to the long-term average, with September being the wettest month of the summer. Favourable water budgets may have contributed to somewhat lower concentrations of the measured parameters, particularly in rivers receiving water from the lakes Eiðisvatn and Hólmavatn. In 2025, tidal influence did not affect measurements in Urriðaá, Kalmansá, or Laxá. However, the sampling locations in these rivers may become inundated during exceptionally high tides. The effects of such events can persist for several days and are typically reflected in elevated sodium and chloride concentrations.

FIGURE 5.1	Freshwater monitoring points in Hvalfjörður in 2025.

Kalmansá and Urriðaá are rock springs whose headwaters originate from surface-water bodies, namely Hólmavatn and Eiðisvatn, respectively.  Laxá, Berjadalsá, Kúludalsá, and Fossá are spring-fed rivers that derive a substantial proportion of their flow from groundwater emerging from the surrounding foothills. Appendix C presents the results of a statistical analysis of chemical measurements from three monitored rivers covering the period 1997–2025. The analysis was conducted to assess whether significant long-term changes in pH and fluoride concentrations have occurred over this period.

### Results

Overall, the results of the year do not indicate major changes in the pollution level of river surface water as compared with the outcome in recent years. The situation has been similar since 2006 [5].

Since monitoring began, all measured values have remained well below the limits established in the Drinking Water Regulation. However, since 2006, somewhat elevated concentrations have been observed in rivers originating from the lakes north of Grundartangi, Eiðisvatn and Hólmavatn.

Detailed freshwater monitoring results are presented in Appendix C. Results from 1998–2015 can be found in previous annual environmental monitoring reports, available from the Icelandic Environment and Energy Agency (in Icelandic).

    - Freshwater Conductivity

Figure 5.2 presents conductivity measurements from the monitored rivers for the period 2016–2025, together with background values from 1997 (and from 2000 for Fossá). Conductivity reflects the concentration of dissolved ions in the water; consequently, higher conductivity values are expected in Kalmansá and Urriðaá, both of which originate from surface-water bodies.

Conductivity in freshwater

300

200

µS/cm

100

0

1997  2000	2016  2017  2018  2019  2020  2021  2022  2023  2024  2025

Kalmansá	Urriðaá	Fossá	Berjadalsá	Laxá	Kúludalsá

FIGURE 5.2	Average conductivity in the monitored rivers during the last 10 years compared with background values from 1997 (2000 for Fossá).

In 2025, the mean conductivity across all six rivers was 128 µS/cm, the highest average value recorded since monitoring began in Kúludalsá in 2016. In Urriðaá and Kalmansá, conductivity ranged from 153 to 271 µS/cm, with an average of 197 µS/cm, representing the highest annual average measured in these rivers since 2019. In the remaining rivers, conductivity ranged from 80 to 120 µS/cm, with an average of 94 µS/cm. Measurements taken on 9 July 2025 yielded the highest conductivity values recorded since monitoring began in Fossá (120 µS/cm), Berjadalsá (103 µS/cm), and Kúludalsá (108 µS/cm).

    - Freshwater pH

The average pH measured in the monitored rivers during 2016–2025 is shown in Figure 5.3. Statistical analysis indicates no significant change in river pH relative to the baseline measurements from 1997 or 2007 (Appendix C).

Freshwater pH

9.5

The Icelandic Drinking Water Regulation pH 6.5 - 9.5

pH

8.5

7.5

6.5

1997 2000	2007	2016 2017 2018 2019 2020 2021 2022 2023 2024 2025

Kalmansá	Urriðaá	Fossá	Berjadalsá	Laxá	Kúludalsá

FIGURE 5.3	Average pH in the monitored rivers during 2016–2025 compared with baseline values from 1997 (2000 for Fossá).

As in previous years, all pH measurements recorded in 2025 were within the drinking water quality limits (pH 6.5–9.5) specified in Regulation No. 536/2001. In Kalmansá and Urriðaá, pH values ranged from 7.06 to 7.70, with an average of 7.35, consistent with the averages observed in recent years. In the remaining rivers, pH values ranged from 6.65 to 7.66, with an average of 7.38. A pH of 6.65 measured in Kúludalsá on 9 July 2025 was the lowest value recorded in that river since monitoring began and the lowest individual pH measurement observed across the monitoring program since 2000. The overall mean pH of all monitored rivers was 7.37, comparable to values recorded in recent years.

It should be noted that samples from Fossá and Laxá were collected only in mid-summer and late summer. As pH values generally increase as the summer progresses, the timing of sampling may influence the annual average values reported for these rivers.

    - Fluoride in Freshwater

Urriðaá and Kalmansá originate from the lakes Eiðisvatn and Hólmavatn, respectively. These lakes are located relatively close to the industrial area at Grundartangi, which can influence fluoride concentrations in the rivers. The southern part of lake Eiðisvatn is situated closest to the industrial area and is therefore most likely to be affected by atmospheric inputs from that source. Fluoride concentrations are also influenced by hydrological conditions, including the water volume of both the rivers and their source lakes. Consequently, year-to-year variations in fluoride concentrations may result from differences in precipitation and runoff patterns, and do not necessarily indicate changes in emissions from the industrial area.

1,500

1,200

The Icelandic Drinking Water Regulation

Fluoride in Freshwater

Kalmansá	Urriðaá

µg F/L

900

600

300

0

1997

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

FIGURE 5.4	Average fluoride concentrations in Urriðaá and Kalmansá during the last 10 years, compared with baseline values from 1997.

In 2025, the average fluoride concentration in Urriðaá was 147 µg F/L, which is consistent with the average recorded during 2016–2024. In Kalmansá, the average concentration was 167 µg F/L, exceeding the average for the same period (Figure 5.4). Fluoride concentrations showed greater variability in 2025 than in recent years, particularly in Kalmansá. Concentrations in this river ranged from a low of 99 µg F/L in April to a high of 220 µg F/L in July. Although fluoride concentrations in both Urriðaá and Kalmansá remain significantly higher than the baseline values measured in 1997, no statistically significant change has been observed relative to 2007. Further details of the statistical analysis are provided in Appendix C.

In the direct runoff streams, the average fluoride concentration was 31 µg F/L, with values ranging from 22 to 40 µg F/L. This corresponds to approximately 20% of the average concentration measured in Urriðaá and Kalmansá. Fluoride concentrations in these rivers are consistent with those observed in recent years.

No environmental quality standard has been established for fluoride concentrations in Icelandic rivers and lakes. However, Regulation No. 536/2001 sets a maximum fluoride concentration of 1,500 µg F/L for drinking water. Since monitoring began, fluoride concentrations in all rivers have remained below 300 µg F/L, and the annual average concentration in any individual monitoring river has never exceeded 200 µg F/L. Studies conducted in the Columbia River basin in North America have indicated that fluoride concentrations exceeding 200 µg F/L in freshwater may adversely affect the migratory behavior and spawning success of adult Pacific salmon [6].

    - Sulfate in freshwater

The highest sulfate (SO4) concentration in freshwater recorded in 2025 was 12.3 mg SO4/L in Kalmansá. This value is well below the maximum concentration of 250 mg SO4/L specified for drinking water in Regulation No.  536/2001.

The average sulfate concentration in Urriðaá and Kalmansá was 7.6 mg SO4/L compared with 6.7 mg SO4/L in the previous year (Figure 5.5).  In the direct runoff streams, the average sulfate concentration was 2.8 SO4/L with individual measurements ranging from 1.5 to 6.3 mg SO4/L. Overall, sulfate concentrations in these rivers were slightly below the long-term average in 2025.

Sulfate in Urriðaá

250

200

The Icelandic Drinking Water Regulation

mg SO₄/L

150

100

50

0

1997	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

FIGURE 5.5	Average sulfate concentrations in Urriðaá and Kalmansá during the last 10 years, compared with baseline values from 1997.

# Seawater monitoring at the flow pits

Two flow pits are located within the harbor area adjacent to the industrial site, where industrial operators at Grundartangi are permitted to dispose of certain types of by-products. In 2025, Norðurál and Elkem, in cooperation with Faxaflóahafnir, constructed a new flow pit west of the existing one (Figure 6.2). The flow pits are used for by-products that cannot be reused or recycled, including spent pot lining, non-saleable metal scrap, refractory materials, and dust from carbon and alumina handling. To prevent solid waste from entering the marine environment, the pits are enclosed by rock embankments that allow seawater to flow through in response to tidal fluctuations. Crushed shell sand is mixed with spent pot lining to neutralize fluoride compounds and reduce the environmental impact of other potentially harmful substances. Faxaflóahafnir’s harbor security is responsible for controlling access to the flow pits and for their day-to-day supervision.

Monitoring of seawater quality forms part of the environmental surveillance program for the Grundartangi industrial area. During the summer of 2025, seawater samples were collected on three occasions at 10 locations along the shoreline and adjacent to the flow pits following high tide. The sampling locations are shown in Figure 6.1. At each location, samples were collected at a depth of approximately one meter and at distances ranging from one to four meters from the rock embankments. In addition, two reference samples were collected, one from the center of Hvalfjörður and the other from Kalastaðir. These reference samples are assumed to represent natural background concentrations of the monitored substances in seawater. Construction activities associated with the new flow pit had minimal impact on the sampling programme during the summer of 2025. Seawater samples were analyzed for arsenic (As), chromium (Cr), copper (Cu), nickel (Ni), lead (Pb), zinc (Zn), iron (Fe), aluminum (Al), phosphorus (P), and vanadium (V), as well as free and total cyanide (CN).

FIGURE 6.1	Location of the seawater sampling stations (photograph by Emil Þór Sigurðsson; image courtesy of Faxaflóahafnir).

Seawater samples were collected on 26 May, 15 July, and 28 July 2025 [7].

All samples intended for fluoride analysis were lost during laboratory handling and could therefore not be analyzed. In addition, six samples collected on 28 July 2025 for other chemical analyses failed during processing. These samples were from stations 1, 6, 8, 9, and 10, as well as the reference station at Kalastaðir.

IMAGE 6.2	Location of the seawater sampling stations (photograph courtesy of Faxaflóahafnir).

### Results

The monitoring results indicate that contamination outside the flow pits remains negligible. Measured concentrations in 2025 were generally comparable to those observed in recent years. In most cases, concentrations were below levels considered likely to affect sensitive ecosystems and below the limits specified in Norðurál’s operating permit. [7]

    - Metals and Other Elements in Seawater Samples

At elevated concentrations, arsenic, lead, copper, chromium, nickel, and zinc are considered undesirable in the marine environment. Environmental quality standards for these substances in surface waters, estuarine waters, and coastal waters are defined in Regulation No. 796/1999 on the Prevention of Water Pollution (Table 6.2). Concentrations of aluminum and fluoride were assessed against the effluent limits specified in Norðurál’s operating permit. No environmental limit values have been established for vanadium, iron, phosphorus, or cyanide in seawater. Average concentrations of all measured substances are presented in Table 6.1, while Appendix D contains average, minimum, and maximum values for all analyzed parameters.

Trace metal concentrations were generally similar to those recorded in recent years. Arsenic, lead, copper, chromium, nickel, and zinc were measured at all sampling stations within Environmental Quality Class II (low risk of environmental impact), indicating minimal leaching of these metals from the flow pits. The highest aluminum concentration was measured in the reference sample collected at Kalastaðir and exceeded the effluent limit of 20 mg/L specified in Norðurál’s operating permit. Iron concentrations were also highest at Kalastaðir. Elevated concentrations of iron and aluminum at this location are likely associated with the corrosion and breakdown of metallic debris present on the seabed near the sampling station. Phosphorus and vanadium concentrations were comparable to those measured at the reference stations.

    - Cyanide in seawater samples

Table 6.1 presents the average concentrations of cyanide and fluoride in seawater samples. Free cyanide concentrations were below the analytical detection limit at all sampling stations (<0.001 mg/L). Total cyanide concentrations were generally at or below the detection limit (<0.001 mg/L); however, two sampling locations, the eastern end of the 1 m section and the center of the 1 m section—showed concentrations slightly above the detection limit. The highest total cyanide concentration recorded was 0.011 mg/L at the eastern end of the 1 m section. No environmental quality standards have been established for cyanide in seawater. For reference, Regulation No. 536/2001 on Drinking Water specifies a limit value of 0.05 mg/L for free cyanide.

.

TABLE 6.1	Average concentrations of substances measured in seawater samples in 2025. Colors indicate the applicable environmental quality classes No environmental quality standards have been established for parameters shown without color coding.

| Monitoring site | As µg/L | Cr µg/L | Cu µg/L | Ni µg/L | Pb µg/L | Zn µg/L | Al* µg/L | P mg/L | Fe** µg/L | P** µg/L | V** µg/L | Cyanide*** (Free) mg/L | Cyanide*** (Total) mg/L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Eastern end - 1 m | 1.75 | 0.35 | <0.5 | <0.5 | <0.3 | <2 | 14.0 | Samples failed in treatment at the analysis agency | 27 | <40 | 2.31 | <0.0010 | 0.0040 |
| 2 Eastern end - 4 m | 1.62 | 0.51 | <0.5 | 0.96 | <0.3 | <2 | 11.3 |  | 38 | <40 | 2.27 | <0.0010 | <0.0010 |
| 3 Eastern side - 1 m | 1.63 | 0.35 | <0.5 | <0.5 | <0.3 | <2 | 13.3 |  | 50 | <40 | 2.28 | <0.0010 | 0.0015 |
| 4 Eastern side - 4 m | 1.74 | 0.27 | <0.5 | <0.5 | <0.3 | <2 | 15.0 |  | 48 | <40 | 2.16 | <0.0010 | 0.0013 |
| 5 Center - 1 m | 1.72 | 0.57 | 0.65 | <0.5 | <0.3 | <2 | 23.3 |  | 65 | <40 | 2.30 | <0.0010 | 0.0067 |
| 6 Center - 4 m | 1.83 | 0.26 | 0.60 | <0.5 | <0.3 | <2 | 15.3 |  | 36 | <40 | 2.18 | <0.0010 | <0.0010 |
| 3 Western side - 1 m | 2.06 | 0.30 | 0.77 | <0.5 | <0.3 | <2 | 22.7 |  | 77 | <40 | 2.25 | <0.0010 | <0.0010 |
| 8 Western side - 4 m | 0.72 | 0.16 | 0.80 | <0.5 | <0.3 | <2 | 13.7 |  | 38 | <40 | 2.26 | <0.0010 | <0.0010 |
| 9 Western end - 1 m | 1.70 | 0.40 | 0.80 | <0.5 | <0.3 | <2 | 14.0 |  | 44 | <40 | 2.26 | <0.0010 | <0.0010 |
| 10 Western end - 4 m | 1.98 | 0.27 | 0.75 | <0.5 | <0.3 | <2 | 12.3 |  | 39 | <40 | 2.34 | <0.0010 | <0.0010 |
| Kalastaðir | 1.96 | 0.28 | 0.65 | <0.5 | <0.3 | 3.11 | 85.0 |  | 179 | <40 | 2.84 | <0.0010 | <0.0010 |
| Center of fjord | 1.89 | 0.22 | <0.5 | <0.5 | <0.3 | <2 | 6.7 |  | 13 | <40 | 2.34 | <0.0010 | <0.0010 |

*Limits specified in Norðurál’s operating permit for cooling water discharged to the sea: aluminum < 20 mg/L and fluoride ≤ 50 mg/L. Fluoride analyses could not be completed because the samples were compromised during laboratory processing.

*No environmental quality standard is available.

***For reference, the maximum permissible concentration of free cyanide in drinking water is 0.05 mg/L, as specified in Directive 98/83/EC and implemented in Iceland through Regulation No. 536/2001 on Drinking Water.

TABLE 6.2	Environmental quality standards for metals in surface freshwaters, transitional waters, and coastal waters according to Regulation No. 796/1999. Colors indicate Environmental Quality Classes I–V.

| Environmental limits for metals Cf. Reg. No. 796/1999 | I µg/L | II µg/L | III µg/L | IV µg/L | V µg/L |
| --- | --- | --- | --- | --- | --- |
| Copper (Cu) | <0.5 | 0.5-3 | 3-9 | 9-45 | >45 |
| Zinc (Zn) | <5 | 5-20 | 20-60 | 60-300 | >300 |
| Lead (Pb) | <0.2 | 0.2-1 | 1-3 | 3-15 | >15 |
| Chromium (Cr) | <0.3 | 0.3-5 | 5-15 | 15-75 | >75 |
| Nickel (Ni) | <0.7 | 0.7-15 | 15-45 | 45-225 | >225 |
| Arsenic (As) | <0.4 | 0.4-5 | 5-15 | 15-75 | >75 |

Very low or no risk of environmental impact

Low risk of environmental impact

Potential impact on sensitive ecosystems

Environmental impacts expected

Water quality consistently unsuitable for aquatic ecosystems

# Vegetation

Vegetation monitoring in 2025 was conducted using the same methods as in previous years. Fluoride concentrations were measured both in plant tissue and in dust deposited on vegetation surfaces. Samples were collected from a range of vegetation types, including grasses, deciduous leaves (birch and rowan), and conifers (spruce and Lodgepole pine). The fluoride concentrations presented in this report represent the total fluoride content, including both fluoride accumulated within plant tissues and fluoride deposited on plant surfaces. Sampling was scheduled to coincide with the growing season, and efforts were made to collect samples following periods of dry weather to minimize the influence of recent rainfall. Vegetation samples were collected at 13 locations in Hvalfjörður (Figure 7.1), with additional reference samples obtained from Skorradalur. In 2025, rowan sampling was introduced at Kalastaðakot. At Gröf II, samples were collected from two locations: one adjacent to the road and another near the farmhouse.

FIGURE 7.1	Vegetation monitoring locations in Hvalfjörður in 2025.

Grass and leaf samples were collected on 5 June and 1 September 2025, while an additional grass sampling round was conducted on 7 July 2025. These sampling dates fall within the main growing season, when vegetation growth rates are highest. Conifer samples were collected on 19 November 2025. Sample collection and laboratory analyses were carried out by Matís [8]. A statistical analysis of fluoride concentrations in grass, leaves, and conifer needles was conducted to evaluate whether significant changes had occurred between years (Appendix E). As vegetation species vary in their sensitivity to fluoride exposure, appropriate reference values differ among vegetation types. The relevant assessment criteria are presented in Table 7.1.

TABLE 7.1	Estimated tolerance of different vegetation types to atmospheric fluoride (HF) and fluoride accumulation in plant tissue [9]

| Concentration of fluoride in the atmosphere. (µg HF/m3) | Fluoride concentration in plant tissue (µg F/g) | Expected impact |
| --- | --- | --- |
| 0 - 0.02 | 0 - 10 | Low/background exposure — No observable effects; concentrations considered representative of background conditions. |
| 0.02 - 0.2 | 10 - 30 | None. |
| 0.2 - 0.6 | 30 - 100 | Decline of sensitive species, particularly mosses, lichens, and conifers [10]. An atmospheric HF concentration of approximately 0.3 µg F/m3 over a 5–6-month period may result in fluoride accumulation of about 30 µg F/g in plant tissue [9]. |
| 0.6 - 1.2 | 100 - 200 | Decline of moderately fluoride-tolerant species, including many grass species |
| > 1.2 | 200 | Decline of fluoride-tolerant species, including deciduous trees and heathland vegetation [11,12,13,10]. |

### Results

Local weather conditions can influence the accumulation of fluoride in vegetation. For example, precipitation may reduce fluoride concentrations in vegetation located close to emission sources by washing deposited fluoride from plant surfaces, whereas prevailing wind patterns can increase fluoride accumulation in areas downwind of those sources. In contrast, temperature generally has a limited influence on fluoride uptake and accumulation in vegetation [8]. Detailed vegetation monitoring results are provided in Appendix E. Results from 1998–2015 are available in previous annual environmental monitoring reports (in Icelandic) published by the Icelandic Environment and Energy Agency.

    - Fluoride in grass

In 2025, fluoride concentrations in grass were below the maximum permissible fluoride concentration in animal feed (30 µg F/g) specified in Regulation No. 1206/2021 on undesirable substances in feed. Measured concentrations were also below the estimated fluoride tolerance thresholds for grazing animals [10] (Table 7.2; Figure E.1 in Appendix E).

The highest fluoride concentration in grass, 28.1 µg F/g, was measured at Stekkjarás in September (Table 7.2; Figure 7.2) [8]. By comparison, the highest concentration measured in the reference area at Skorradalur was 3.0 µg F/g.  Average fluoride concentrations in grass increased slightly compared with the previous year but remained well below all relevant guideline and tolerance values. Fluoride concentrations varied among sampling locations and between sampling periods (Table 7.2). Results for individual monitoring stations are presented in Appendix E.

Average fluoride concentrations in grass at monitoring sites both north and south of Hvalfjörður were higher in 2025 than the corresponding baseline values from 1997. However, statistical analysis showed no significant change relative to 2007 for either region. Throughout the monitoring period, fluoride concentrations have generally been higher in grass collected north of the fjord than in samples collected south of the fjord, although these differences have not always been statistically significant, including in 2025. Variability in measured concentrations was comparable on both sides of the fjord.

TABLE 7.2	Results of fluoride measurements in grass from all monitoring stations in 2025, including tolerance thresholds, statistical evaluations, and weather conditions during sampling.

| Total fluoride concentration in grass |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Monitoring site | June (μg F/g dry weight) | July (μg F/g dry weight) | September (μg F/g dry weight) | Change in 2025 relative to 1997 | Change in 2025 relative to 2007 |
| North of the fjord | Stekkjarás | 7.7 | 5.1 | 28.1 | Elevation | No Change |
|  | Fannahlíð | 6.2 | 3.2 | 18.1 |  |  |
|  | Fellsaxlarkot | 5.5 | 4.2 | 14.0 |  |  |
|  | Kalastaðarkot | 2.9 | 3.5 | 6.4 |  |  |
|  | Ferstikla | 1.3 | 3.1 | 3.3 |  |  |
|  | Gröf II by the house | 6.1 | 7.0 | 9.6 |  |  |
|  | Hólabrú | 5.7 | 13.4 | 9.5 |  |  |
|  | Hlíð | 2.2 | 9.7 | 4.8 |  |  |
| South of the fjord | Félagsgarður | 10.3 | 5.5 | 12.9 |  |  |
|  | Fossbrekka | 7.7 | 9.2 | 4.0 |  |  |
|  | Háls in Kjós | 2.4 | 6.8 | 2.0 |  |  |
|  | Reynivellir | 3.2 | 20.4 | 2.0 |  |  |
| Reference site Skorradalur |  | 3.0 | - | 3.0 |  |  |
| Estimated tolerance threshold for grasses Estimated tolerance threshold for grazing animals Maximum fluoride concentration permitted in feed for sheep** |  |  |  |  | 100–200 µg F/g in plant tissue 30 µg F/g in plant tissue 30 µg F/g for sheep |  |

**According to Regulation No. 1206/2021 on undesirable substances in feed, the maximum fluoride concentration for herbivores is 50 µg/g in feed with a moisture content of 12%, and 30 µg/g for feed intended for lactating animals.

| Weather Conditions During Sampling |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | June | July | September | Explanation |
| Date | 5 | 7 | 1 | Spring 2025 was exceptionally warm, and mean summer temperatures exceeded the long-term average. Total precipitation was close to normal. As is typically the case in Hvalfjörður, easterly and northeasterly winds predominated throughout the summer of 2025. No precipitation occurred on any of the sampling dates, and weather conditions were generally dry during the days preceding sampling. |
| Total precipitation during the five days preceding sampling (mm) |  |  |  |  |
|  | 6 |  | 5 |  |
| Outside temperature (°C) | 5.9 |  | 13.2 |  |
| Average wind speed (m/s) | 5 |  | 2 |  |
| Wind direction | NA, NV- |  | A- winds |  |
|  | direction |  |  |  |

30

Reference values for feed

Grass

20

μg F/g

10

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

Average north of Grundartangi	Average east of Grundartangi

Average south of Hvalfjörður	Average west of Grundartangi Skorradalur

FIGURE 7.2	Average fluoride concentration in grass from monitoring sites based on the location at Grundartangi, along with background values ​​from 1997, measurements from 2007, and from Skorradalur as a reference.

    - Fluoride in foliage

Fluoride concentrations in leaves were below the estimated tolerance threshold for deciduous trees (200 µg F/g) at all monitoring sites (Table 7.3; Figure E.2 in Appendix E). The highest fluoride concentration was measured at Stekkjarás in September, reaching 81.1 µg F/g.  By comparison, the average fluoride concentration in leaf samples collected from the reference site at Skorradalur in September was 7.0 µg F/g.

The average fluoride concentration in leaves in 2025 was similar to that recorded in 2024. However, a slight increase was observed at monitoring sites north of Grundartangi, likely reflecting the prevalence of easterly and southeasterly winds during the growing season. Fluoride concentrations also varied throughout the growing season (Table 7.3).

Results from individual monitoring sites are presented in Appendix E. Table 7.3 summarizes the average fluoride concentrations measured in 2025 and the results of the statistical analysis. The analysis showed that average fluoride concentrations in leaves at sites both north and south of Hvalfjörður were higher than the corresponding baseline values from 1997. However, no significant change was detected relative to 2007 (Figure E.3 in Appendix E). Figure 7.3 presents the average fluoride concentrations in leaves for monitoring sites located west, north, and east of Grundartangi, as well as south of Hvalfjörður. In general, greater variability in fluoride concentrations was observed at sites north and west of Grundartangi than at sites east of Grundartangi or south of the fjord.

TABLE 7.3	Results of fluoride measurements in grass from all monitoring stations in 2025, including tolerance thresholds, statistical evaluations, and weather conditions during sampling. Weather conditions during sampling are presented in Table 7.2.

| Total fluoride concentration in leaves |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Monitoring site | Average concentration in June (µg F/g dry weight) | Average concentration in August (µg F/g dry weight) | Change in 2025 relative to 1997 | Change in 2025 relative to 2007 |
| North of the fjord | Stekkjarás | 19.4 | 81.1 | Elevation | No Change |
|  | Fannahlíð | 23.6 | 63.4 |  |  |
|  | Fellsaxlarkot | 17.4 | 43.2 |  |  |
|  | Kalastaðakot | 5.4 | 16.3 |  |  |
|  | Ferstikla | 4.7 | 6.8 |  |  |
|  | Gröf II by the house | 9.5 | 16.9 |  |  |
|  | Hólabrú | 7.6 | 27.7 |  |  |
|  | Hlíð | 7.0 | 18.4 |  |  |
| South of the fjord | Félagsgarður | 10.4 | 23.5 |  |  |
|  | Fossbrekka | 5.5 | 9.0 |  |  |
|  | Háls in Kjós | 6.5 | 14.0 |  |  |
|  | Reynivellir | 14.3 | 22.5 |  |  |
| Reference site Skorradalur |  | 2.5 | 7.0 |  |  |
| Estimated tolerance threshold for deciduous trees |  | 200 µg F/g in plant tissue |  |  |  |

200

150

Estimated tolerance threshold for deciduous trees

Leaves

100

µg F/g

50

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

Average concentration north of Grundartangi	Average concentration east of Grundartangi

Average concentration south of Hvalfjörður	Average concentration west of Grundartangi

FIGURE 7.3	Average fluoride concentrations in leaves at monitoring sites grouped by their location relative to Grundartangi in 2025, together with baseline values from 1997, measurements from 2007, and values from the reference site at Skorradalur.

    - Fluoride in pine needles

The average fluoride concentration in one-year-old conifer needles remained below the estimated tolerance threshold for coniferous trees, which ranges from 30 to 100 µg F/g (Table 7.4). Apart from the monitoring site at Kalastaðakot, average fluoride concentrations were higher in two-year-old needles than in one-year-old needles. Figure 7.4 presents the average fluoride concentrations measured in one- and two-year-old needles across all monitoring sites.

TABLE 7.4	Results of fluoride measurements in conifer needles from all monitoring sites in 2025, together with tolerance thresholds, summaries of the statistical analyses, and weather conditions during sampling.

| Fluoride in conifer needles |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Monitoring site | Average concentration in one-year-old needles (µg F/g dry weight) | Average concentration in two-year-old needles (µg F/g dry weight) | Changes in 2025 relative to 1997 | Changes in 2025 relative to 2007 |
| North of the fjord | Stekkjarás | 12.5 | 24.5 | No Change | No Change |
|  | Fannahlíð | 14.5 | 21.7 |  |  |
|  | Fellsaxlarkot | 5.7 | 6.8 |  |  |
|  | Kalastaðakot* | 1.3 | 0.8 |  |  |
|  | Ferstikla* | 0 | 0 |  |  |
|  | Gröf II by road* | 3.7 | 7.3 |  |  |
|  | Hlíð* | 2.6 | 3.9 |  |  |
|  | Hólabrú* | 4.8 | 10.2 |  |  |
| South of the fjord | Félagsgarður | 0.8 | 1.8 |  |  |
|  | Fossbrekka | 0.5 | 0.8 |  |  |
|  | Háls in Kjós* | 1.4 | 2.4 |  |  |
|  | Reynivellir | 2.3 | 4.2 |  |  |
| Reference site Skorradalur |  | 0.5 | 1.5 |  |  |
| Estimated tolerance threshold for coniferous trees |  | 30-100 µg F/g in plant tissue |  |  |  |

* Monitoring began at Hlíð in 2009, at Gröf II (near the house) in 2010, and at Ferstikla and Háls in Kjós in 2011. Monitoring at Hólabrú commenced in 2018 and at Kalastaðakot in 2022.

| Weather Conditions During Sampling |  |  |
| --- | --- | --- |
|  | November | Explanation |
| Date | 19 | During the days preceding sampling, average temperatures were approximately 0.5°C, and easterly winds predominated. Weather conditions were relatively dry before sampling; however, precipitation occurred on the sampling day, falling as snow during the morning and rain later in the afternoon. |

Pine needles

Tolerance limit of coniferous trees (30 - 100 µg/g)

One–year-old needles

Two–year-old needles

100

80

60

µg F/g

40

20

0

Average concentration in one- and two-year-old needles north of Grundartangi	Average concentration in one- and two-year-old needles east of Grundartangi Average concentration in one- and two-year-old needles south of Hvalfjörður	Average concentration in one- and two-year-old needles west of Grundartangi

FIGURE 7.4	Average fluoride concentrations in one- and two-year-old conifer needles at monitoring sites west, north, and east of Grundartangi, and south of Hvalfjörður, compared with baseline values from 1997 and 2007.

No significant changes in average fluoride concentrations were detected in either one- or two-year-old needles north or south of Hvalfjörður in 2025 relative to the baseline years 1997 or 2007 (Figure 7.4). Fluoride concentrations generally remained higher in conifer needles collected north of the fjord than in those collected south of the fjord. Although this difference was not statistically significant in 2025, significant differences have been observed in some previous years (Figures E.5 and E.6, Appendix E).

# Herbivores (sheep and horses)

Fluoride concentrations are monitored in the jawbones of slaughtered grazing animals, including lambs and adult sheep, from farms located both north and south of Hvalfjörður (Figure 8.1). The jaws are also examined for signs of fluoride-related damage [14]. A veterinarian from the Mosfellsbær Veterinary Clinic conducted examinations of the sheep jaws, while Matís analyzed fluoride concentrations in jawbone ash. In addition, veterinarians inspected live sheep and horses for evidence of fluoride-related dental damage and assessed the condition of the joints in the forelimbs [15].

Grazing animals are exposed to fluoride primarily through feed and drinking water. Fluoride accumulates in bone tissue over time, and concentrations are therefore generally higher in older animals. As part of the monitoring programme, four jawbones from lambs and four jawbones from adult animals (six years of age or older) are collected and analyzed from each participating farm whenever possible [1]. Information on the age of the animals is obtained from farm records.

FIGURE 8.1	Monitoring farms for sheep and horses in 2025.

Jawbone samples from lambs and/or adult sheep slaughtered in autumn 2025 were obtained from 12 farms participating in the monitoring programme (Figure 8.1). Reference samples were collected from Bjarnarhöfn on Snæfellsnes, where sheep are partially grazed on coastal pasture, and from Skjaldfönn in Ísafjarðardjúp. Fluoride concentrations were determined in a total of 95 jawbone ash samples, comprising 49 lambs and 46 adult sheep. No lamb samples meeting the sampling protocol were obtained from Eystra-Miðfell because of collection failures at the slaughterhouse, and no adult sheep samples were received from Vestri-Reynir. South of Hvalfjörður, samples were collected from Miðdalur for the first time, while samples from Fell were included again after last being analyzed in 2007.

Fluoride can accumulate in the bones and teeth of grazing animals through several pathways, including emissions from aluminum smelters, coastal grazing, and the application of fluoride-rich fertilizers [14]. Natural sources of fluoride include volcanic activity, geothermal systems, forest fires, biomass burning, and sea spray [9]. The fluoride tolerance of sheep is not fully understood, as it depends on several factors, including age, diet, productivity, reproductive status, and the chemical form of fluoride involved. Because no specific assessment criteria have been established for fluoride-related dental effects in sheep, the monitoring programme continues to refer to thresholds developed from studies of young Norwegian deer. The criteria originally published in 1996 [16] were subsequently refined through the ESPIAL (Ensuring Sustainable Production of Primary Aluminium) project [17]. The fluoride concentrations in jawbone ash associated with varying levels of risk for dental fluorosis in deer are presented in Table 8.1 [13,17]. Studies indicate that sheep and goats generally exhibit fewer and milder symptoms of fluoride toxicity than many other grazing animals, both in areas with high and low fluoride concentrations in drinking water. This suggests a relatively high tolerance or reduced sensitivity to fluoride exposure, although further research is needed to confirm these findings and identify the underlying causes [18]. Consequently, the fluoride thresholds derived from deer studies are considered conservative when applied to sheep.

TABLE 8.1	Estimated effects of accumulated fluoride in the jaws and teeth of grazing animals, based on studies of 1.5-year-old Norwegian deer [13,17].

| Fluoride concentration in jawbone ash (µg F/g) | Impact |
| --- | --- |
| < 900 | No impact |
| 900 – 2,000 | Considered capable of causing dental fluorosis in deer |
| > 2,000 | Associated with dental fluorosis in deer |

For context, fluoride concentrations of approximately 180–200 µg F/g have been measured in the jawbone ash of 4–5-month-old Icelandic lambs that had not been exposed to elevated fluoride concentrations during the grazing season [19]. Furthermore, measurements conducted between 1968 and 1971 on 25 adult sheep from several regions of Iceland (Gullbringu, Árnes, Borgarfjarðar, and Snæfellsnes counties) yielded an average fluoride concentration of 1,150 µg F/g in jawbone ash, with concentrations ranging from 780 to 2,400 µg F/g \[19]. A British study reported typical fluoride concentrations in the bones of ruminants between 1,000 and 1,500 µg F/g \ [20]. A recent Icelandic study demonstrated a statistically significant relationship between fluoride accumulation in the jawbones of sheep and horses and proximity to an aluminum smelter. The study also showed that fluoride accumulation increases with age until reaching a peak at approximately eight years of age in sheep and eighteen years of age in horses, after which concentrations gradually decline. In both species, the highest rates of accumulation occur during periods of rapid bone and cartilage growth, generally up to three to four years of age. Fluoride uptake into skeletal tissue is greatest while bones are actively growing [21].

Previous studies have further indicated that when fluoride concentrations in grass exceed approximately 30 µg F/g during the growing season, the risk of excessive fluoride accumulation in bones, including the jawbones of sheep, increases, particularly under prolonged exposure conditions [9].

A veterinarian examined the teeth and limb joints of live sheep from six farms located both north and south of Hvalfjörður—Hrafnabjörg, Eystra-Miðfell, Vogatunga, Kiðafell, Miðdalur, and Fell. A total of 59 sheep, ranging in age from one to eight years, were assessed for signs of fluoride-related dental and skeletal effects.

Similarly, 29 horses from five farms—Litla-Fellsöxl, Kalastaðakot, Ytri-Hólmur, Miðdalur, and Meðalfell—were examined for evidence of fluoride-related changes in teeth and joints. Both young and older horses were included, with ages ranging from 3 to 31 years. Preference was given to horses that had spent most of their lives on the monitored farms and were therefore suitable for repeated long-term assessments. Clinical examinations of teeth and joints of live sheep and horses were conducted during January and February 2026.

The severity of fluoride-related dental changes in live grazing animals (sheep and horses) was assessed using the scoring system presented in Table 8.2.

TABLE 8.2    Scoring System for Dental Changes in Live Livestock [15].

| Score | Description |
| --- | --- |
| 0 | Normal |
| 1 | Uncertain effect (e.g. minor staining of the enamel may be visible). |
| 2 | Minor effect. Slight enamel staining or streaking without abnormal wear. Mild discoloration may be present. |
| 3 | Moderate effect. Noticeable staining, widespread enamel dullness, and small pits in the enamel. Increased tooth wear and discoloration may be evident. |
| 4 | Significant effect. Extensive enamel dullness, some underdevelopment of teeth resulting in smaller teeth, enamel pitting, increased wear, and likely staining. |
| 5 | Severe effect. Marked tooth underdevelopment, advanced wear, enamel pitting and erosion, and pronounced staining. |

### Results

The average fluoride concentrations measured on monitoring farms around Hvalfjörður, together with estimated tolerance thresholds for herbivores, are summarized in Table 8.3. The table also presents changes in fluoride concentrations in 2025 relative to 1997, when monitoring began, and to 2007, when all smelter production lines were fully operational.

TABLE 8.3	Monitoring results for sheep and estimated tolerance levels of herbivores for fluoride concentration in bones, as well as statistical analysis results.

| Monitored farms | Average fluoride concentration in jawbone (µg F/g dry matter) | Changes in 2025 relative to 1997 | Changes in 2025 relative to 2007 |
| --- | --- | --- | --- |
| Lambs |  |  |  |
| Farms north of Hvalfjörður |  |  |  |
| Farm east of the industrial area (Hrafnabjörg) | 124 |  |  |
| Farms west/southwest of the industrial area (Vestri Reynir, Innri Hólmur) | 237 |  |  |
| Farms north/northwest of the industrial area (Vogatunga, Eystri Leirárgarðar, Skipanes, Skorholt, Hóll) | 113 |  |  |
| Alls farms north of Hvalfjörður | 136 | Elevation | Decrease |
| Farms south of Hvalfjörður (Kiðafell, Miðdalur, Fell) | 130 | Elevation | Decrease |

| Adult sheep |  |  |  |
| --- | --- | --- | --- |
| Farms north of Hvalfjörður |  |  |  |
| Farm east of the industrial area (Hrafnabjörg) | 861 |  |  |
| Farm west/southwest of the industrial area (Innri Hólmur) | 1,186 |  |  |
| Farms north/northwest of the industrial area (Vogatunga, Eystri Leirárgarðar, Skipanes, Skorholt, Hóll, Eystra Miðfell) | 925 |  |  |
| All farms north of Hvalfjörður | 925 | Elevation | No Change |
| Farms south of Hvalfjörður (Kiðafell, Miðdalur, Fell) | 803 | Elevation | Elevation |
| Criteria |  |  |  |
| Considered risk of dental fluorosis in deer | 900 - 2,000 |  |  |
| Associated with dental fluorosis in deer | >2,000 |  |  |

Appendix F presents average fluoride concentrations in the jawbones of adult sheep and lambs from all monitoring farms, together with detailed monitoring results, statistical analyses, and summaries of veterinary examinations of teeth and joints in live horses and sheep. Results from earlier monitoring years (1998–2015) are reported in previous annual environmental monitoring reports, which are available from the Icelandic Environment and Energy Agency (in Icelandic).

    - Fluoride in Lambs

Fluoride concentrations in the jawbones of lambs were below the threshold associated with a risk of dental fluorosis in deer (900 μg F/g dry weight) at all monitoring locations (Appendix F.1). The highest average concentration was measured in lambs from Innri Hólmur (275 μg F/g), while the lowest concentrations were recorded in lambs from Eystri Leirárgarðar and Kiðafell (89 μg F/g).

The overall average fluoride concentration in lamb jawbones across all monitoring farms was 134 μg F/g, comparable to levels measured in recent years. Average concentrations were similar on farms north of Hvalfjörður (136 μg F/g) and south of the fjord (130 μg F/g) (Figure 8.2). For comparison, fluoride concentrations measured in reference samples were 102 μg F/g at Skjaldfönn and 60 μg F/g at Bjarnarhöfn. Statistical analysis showed a significant increase in average fluoride concentrations compared with 1997. However, concentrations were lower than those measured in 2007 (Appendix F.2).

In 2025, average fluoride concentrations in lambs from areas east, north, northwest, and south of the industrial area remained within the range observed since 2008.

1000

Lambs - all monitored farms

Considered capable of causing dental fluorosis in deer

µg F/g

500

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

Average concentration of all farms north of Hvalfjörður	Average concentration of all farms south of Hvalfjörður

Skjaldfönn, N-Ísafjörður county	Bjarnarhöfn, Snæfellsnes

FIGURE 8.2	Average fluoride concentration in the jawbones of lambs from farms north and south of the fjord, plus reference samples from 1997 and 2007.

    - Fluoride in adult livestock

Because fluoride accumulates in bone over time, direct year-to-year comparisons of fluoride concentrations in adult sheep are complicated by differences in age structure. Icelandic studies have demonstrated a positive relationship between age and fluoride accumulation in the jawbone, although accumulation generally slows after approximately eight years of age [21]. The monitoring program aims to sample sheep aged six years or older, although this target has not always been achieved. In 2025, the average age of sampled sheep was approximately 6.2 years, ranging from 3 to 9 years.

No animal exceeded the fluoride concentration considered capable of causing dental fluorosis in deer (2,000 μg F/g). The highest average fluoride concentration was measured in sheep from Eystra Meðalfell (1,336 μg F/g; range 862–1,740 μg F/g). Average concentrations also exceeded the deer fluorosis risk threshold of 900 μg F/g (see Appendix F) at several farms, including: Skipanes: 1,075 μg F/g (960–1,205 μg F/g), Innri Hólmur: 1,186 μg F/g (single animal), Vogatunga: 1,040 μg F/g (805–1,296 μg F/g), Skorholt: 946 μg F/g (774–1,114 μg F/g). The lowest average fluoride concentration was measured at Eystri Leirárgarðar (602 μg F/g). Reference samples contained substantially lower concentrations, averaging 256 μg F/g at Skjaldfönn (animals aged 7–9 years) and 112 μg F/g at Bjarnarhöfn (animals aged 4–7 years).

Across all monitoring farms, the average fluoride concentration in adult sheep jawbones was 893 μg F/g.  Average concentrations were 925 μg F/g on farms north of the fjord and 803 μg F/g on farms south of the fjord (Figure 8.3). The average concentration measured in 2025 fell within the range observed since 2008.

Since monitoring began in 1997, fluoride concentrations have generally been higher in adult sheep north of Hvalfjörður than south of the fjord. In 2025, however, the difference was not statistically significant, although significant differences have been observed in some previous years (Appendix F.2).

Compared with 2007, no significant change was detected in average fluoride concentrations in adult sheep north of Hvalfjörður. However, concentrations were significantly higher than in 1997. South of the fjord, average fluoride concentrations were significantly higher than those measured in both 1997 and 2007.

2000

Associated with dental fluorosis in deer

Adult sheep - all monitoring farms

1500

Considered capable of causing dental fluorosis in deer

µg F/g

1000

500

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

Average concentration of all farms north of Hvalfjörður	Average concentration of all farms south of Hvalfjörður Bjarnarhöfn, Snæfellsnes	Skjaldfönn, N-Ísafjarðarsýsla

FIGURE 8.3	Average fluoride concentration in the jawbones of adult sheep from farms north and south of Hvalfjörður, plus reference samples from 1997 and 2007.

    - Examination of jaws and teeth in slaughtered sheep

A veterinarian examined jaw samples from slaughtered sheep to assess visible signs of fluoride-related effects on teeth and bone. Jaws from both lambs and adult sheep collected at monitoring farms and reference sites were assessed for

enamel condition, tooth wear, tooth loss, discoloration, enamel defects, and jawbone condition. An overview of the veterinary assessment criteria is presented in Table 8.4. Detailed inspection records and summaries of the findings are provided in Appendix F.3 [14].

TABLE 8.4    Overview of a veterinarian's assessment of the dental health of slaughter animals.

| Dental health of lambs* | Farms by Hvalfjörður Number of animals / average concentration (μg F/g dry material) | Reference farms Number of animals / average concentration (μg F/g dry material) |
| --- | --- | --- |
| Good | 41 / 134 | 8 / 81 |
| Fair | 0 | 0 |
| Poor | 0 | 0 |
| Dental health of adult sheep* | Farms by Hvalfjörður Number of animals / average concentration (μg F/g dry material) | Reference farms Number of animals / average concentration (μg F/g dry material) |
| Good | 11 / 910 | 1 / 94 |
| Fair | 6 / 1,010 | 0 |
| Poor | 21 / 851 | 6 / 197 |
| Average age (years) | 6.2 | 6.6 |

* Good: Teeth show no signs of abnormalities or damage. Fair: Minor imperfections are present but are unlikely to affect the animal's health or wellbeing during its lifetime. Poor: Dental condition is compromised and may adversely affect the animal's health or function.

All lambs examined were assessed as having good dental health. In contrast, poor dental health was relatively common among adult sheep. The highest fluoride concentration measured in a lamb was 279 μg F/g in an animal from Innri Hólmur. Sixteen adult sheep had fluoride concentrations above the threshold associated with an increased risk of dental fluorosis in deer (900 μg F/g), compared with twelve animals in 2024. Despite these elevated fluoride concentrations, affected animals were classified across all categories of dental health—good, fair, and poor. Changes in tooth enamel were observed in three lambs and one adult sheep.

Although dental condition generally declined with age, the results did not indicate a clear relationship between fluoride concentration in the jawbone and poor dental health [14].

    - Inspection of teeth and joints in live sheep and horses

A veterinarian examined the teeth and forelimb joints of live sheep and horses to assess potential effects of fluoride exposure. The results indicated no detectable fluoride-related effects. Dental condition was generally within the range considered normal by the examining veterinarian, and no fluoride-associated changes were identified in the joints of either species.

Appendix F.3 provides a detailed summary of the examinations of teeth and forelimb joints in live sheep and horses, based on the scoring system presented in Table 8.2. Overall, scores for both dental and joint conditions were low, indicating little evidence of abnormalities. One horse received a dental score of 3, corresponding to a moderate effect, due to visible pitting or erosion of the enamel. However, it could not be determined whether these changes were caused by fluoride exposure, and similar findings were not observed in any other horse. Alternative causes, such as tartar accumulation, may also contribute to tooth and gum damage. The highest average dental condition score was recorded in horses from Litla Fellsöxl (0.26), while the lowest was recorded in horses from Kalastaðakot (0.07). No abnormalities were observed in the joints of any horses examined.

Among the sheep, a small number of missing or loose incisor teeth were observed. Osteoarthritis of the jaw was identified in two animals, likely originating from the molar region. No abnormalities were detected in sheep joints, although a minor swelling was noted in the knee of one sheep from Eystra Miðfell [15].

# References

- Environmental Monitoring Plan for the Grundartangi Industrial Area 2018–2028,  Second Revision	28 March 2018,“	2018. (In Icelandic)	[Available	online].	Available: https://ust.is/library/Skrar/Atvinnulif/Starfsleyfi/Eftirlitsskyrslur/alver/Voktunaraaetlu n_2018_2021_2endursk_28032018.pdf.

- Hvalfjarðarsveit. “Hvalfjarðarsveit Master Plan 2020–2032,” 2023. (In Icelandic)

- Leandro Pellegrini, „Environmental Monitoring Plan for the Grundartangi Industrial Area 2018–2028, Measurements of Atmosphere and Precipitation. January-December 2025,” Matís, 2026

- Icelandic Meteorological Office, “Sulfur pollution - measurements for decades,” no year.  [Online]. Available here in Icelandic: http://www.vedur.is/mengun/mengun/brennisteinn. [Accessed in May 2026].

- Egill	Antonsson,	“Environmental monitoring of the industrial area of Grundartangi - Freshwater measurements in 2025, “ Hafrannsóknastofnun, Hafnarfjörður, 2026. (In Icelandic)

- Camargo, Julio A., “Fluoride toxicity to aquatic organisms: a review, “ Chemosphere,

b. 50, No. 3, pp. 251-264, 2003.

- Egill Antonsson, “Environmental monitoring of the industrial area of Grundartangi.  Measurements in the sea at the flow pits 2025,“ 6EM24003, Chemical Analysis, Hafrannsóknastofnun, Hafnarfjörður, 2026. (In Icelandic)

- Egill Antonsson, “Environmental monitoring of the industrial area of Grundartangi, Measurements in vegetation 2025,“ Matís, 2026. (In Icelandic)

- R. Liteplo, R. Gomes, P. Hower og H. Malcolm, “Fluorides. Environmental Health Criteria 227,“ World Health Organization (WHO), 2002.

- Weinstein L.H. and Davison A.W., “Fluorides in the Environment. Effects on Plants and Animals,“ CABI Publishing, Wallingford, Great Britain, 2004.

- Pálmason F, and E. Skye, “An evaluation of the impact of airborne emissions from a planned aluminium smelter on the vegetation in Reyðarfjörður. A report to the Icelandic energy marketing agency,“ RALA, Reykjavík, 1999.

- Horntvedt and Øyen, “Effekter af fluorider på skog ved norske aluminiumsverk.” (The effects of fluoride in the vicinity of Norwegian smelters) Chapter 6 in Norsk aluminiumindustri og miljø. Project for effecstudier av industriutslipp fra primæraluminiumverk i Norge.,“ Aluminiumindustriens Miljøsekretariat (AMS), Oslo, 1994. (In Norwegian)

- Ongstad, Stoll and Aasland, “The Norwegian aluminium industry and the local environment,“ Project to study the effects of industrial emission from primary aluminium plants in Norway. Summary report, 1994.

- Þórunn Lára Þórarinsdóttir, Veterinarian Mosfellsbær, “Fluorometry of jawbones and dental examination in sheep for the year 2025. Report on environmental monitoring of the industrial area in Grundartangi,” Veterinarian Mosfellsbær, Mosfellsbær, 2025. (In Icelandic)

- Þórunn Lára Þórarinsdóttir, Veterinarian Mosfellsbær, “Report for the year 2025, which is part of the environmental monitoring of the industrial area in Grundartangi 2018-2028. Livestock,” Veterinarian Mosfellsbær, Mosfellsbær, 2026. (In Icelandic)

- T. Vikøren and G. Stuve, “Fluoride Exposure in Cervids Inhabiting Areas Adjacent to Aluminum Smelters in Norway. II. Fluorosis,“ Journal of Wildlife Diseases, b. 32, No. 2, pp. 181-189, 1996.

- T. Vikøren and G. Stuve, “ESPIAL Fauna - Current state for fluoride exposure of

animals in the vicinity of aluminium smelters,“ 2021.

- S. L. Choubisa, “Are Sheep and Goat Animals Relatively More Tolerant to Fluorosis?“ Journal of Veterinary Medicine and Research, 2024.

- Páll A. Pálsson, “Fluoride pollution and aluminum smelters, fluorine levels in animal bones near the aluminum smelter in Straumsvík in the years 1967-1991,“ Búnaðarrit nr. 108, pp. 245-257, 1995. (In Icelandic)

- C. Livesey og J. Payne, “Diagnosis and investigation of fluorosis in livestock and horses,“ In Practice, b. 33, No. 9, pp. 454-461, 2011.

- C. Oddsdóttir, E. Sturludóttir, Ó. G. Sigurðardóttir, B. Valgeirsdóttir, E. I. Eyjólfsdóttir and B. Sigríður, “Accumulation of fluoride in Icelandic sheep and horses 2007-2019 and the effect of proximity to aluminium smelters,“ 2023.

- National Research Council, “Effects of Fluorides in Animals,“ National Academy of

Science, National Academies Press, 1974.

# Appendix A	Older environmental monitoring reports

Joint environmental monitoring of the Grundartangi industrial area has been conducted since 1999. Previously published annual environmental monitoring reports are as follows and contain information on the results of the monitoring since its inception (all in Icelandic):

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2024. May 2025.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2023. May 2024.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2022. May 2023.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2021. June 2022.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2020. April 2021.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2019. April 2020.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2018. May 2019.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2017. April 2018.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2016. April 2017.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2015. April 2016.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2014. April 2015.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2013. April 2014.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2012. April 2013.

- Environmental Monitoring Plan for the Grundartangi Industrial Area Results of the Year 2011. April 2012.

- Environmental monitoring of the industrial plants at Grundartangi. Results of the Year 2010. April 2011.

- The Industrial Area at Grundartangi. Environmental monitoring results for the 2009 operating year. April 2010.

- The Industrial Area at Grundartangi. Environmental monitoring results for the 2008 operating year. April 2009.

- The Industrial Area at Grundartangi. Environmental monitoring results for the 2007 operating year. May 2008.

- The Industrial Area at Grundartangi. Environmental monitoring results for the 2006 operating year. October 2007.

- The Industrial Area at Grundartangi and its Vicinity. Environmental monitoring results for 2005. November 2006.

- The Industrial Area at Grundartangi and its Vicinity. Environmental monitoring results for 2004. Final report. April 2005.

- The Industrial Area at Grundartangi and its Vicinity. Environmental monitoring results for 2003. Final report. April 2004.

- The Industrial Area at Grundartangi and its Vicinity. Environmental monitoring results for 2002. Final report. March 2003.

# Appendix B	Air quality

TABLE B.1  Results of weather and air quality measurements at Kríuvarða in 2025, average concentration and highest and lowest daily and hourly values ​​of the year, along with defined environmental limits that apply outside the industrial area.

| Parameters | Concentration (µg/m3) | Reference value (µg/m3) | Explanation |
| --- | --- | --- | --- |
| F (filters, Apríl - October) |  |  |  |
| Average HF concentration | 0.36 |  |  |
| Average HF concentration (April - September) | 0.38 |  | Within the Industrial Area at Grundartangi. |
| Highest measured average HF concentration | 2.87 |  | Measured 9 May in south-south-western wind |
| Lowest measured average HF concentration | 0.00 |  |  |
| Average total HF concentration | 0.41 |  |  |
| Average total HF concentration (April - September) | 0.43 |  | Within the Industrial Area at Grundartangi. |
| Highest measured average total HF concentration | 3.38 |  | Measured 9 May in south-south-western wind |
| Lowest measured average total HF concentration | 0.00 |  |  |
| F (filters, throughout the year) |  |  |  |
| Average HF concentration | 0.27 |  |  |
| Highest measured average HF concentration | 2.87 |  | Measured 9 May in south-south-westerly wind |
| Lowest measured average HF concentration | 0.00 |  |  |
| Average total HF concentration | 0.31 |  |  |
| Highest measured average total HF concentration | 3.38 |  | Measured 9 May in south-south-westerly wind |
| Lowest measured average total HF concentration | 0.00 |  |  |
| SO2 |  |  |  |
| Average annual concentration (gaseous, in filters) | 3.35 |  |  |
| Annual average (automatic measurements) | 4.30 | 20 | Vegetation protection limit cf. Regulation 920/2016. |
| Highest daily mean value of the year | 82 | 125 | Vegetation protection limit cf. Regulation 920/2016. Allowed to exceed 3x over the year. Measured 15 January in south-south-westerly wind |
| Lowest daily mean value of the year | 0.0 |  |  |
| Highest hourly mean value of the year | 160 | 350 | Vegetation protection limit cf. Regulation 920/2016. Allowed to exceed 24x over the year. Measured 21 July in northwesterly wind |
| H2S |  |  |  |
| Average concentration over the year | 0.4 | 5 | Vegetation protection limit cf. Regulation 514/2010. |
| Highest daily mean value of the year | 2.2 | 50 | Vegetation protection limit cf. Regulation 514/2010. Allowed to exceed 5x over the year. Measured 21 August in east-southeasterly wind |
| Lowest daily mean value of the year | 0.0 |  |  |
| NO2 |  |  |  |
| Average concentration over the year | 0.79 | 40 | Vegetation protection limit cf. Regulation 920/2016. |
| Highest daily mean value of the year | 12.9 | 75 | Vegetation protection limit cf. Regulation 920/2016. Allowed to exceed 7x over the year. Measured 30 August in easterly wind |
| Lowest daily mean value of the year | 0.0 |  |  |
| Highest hourly mean value of the year | 30.1 | 200 | Vegetation protection limit cf. Regulation 920/2016. Measured 30 August in northerly wind |
| NOx |  |  |  |
| Average concentration over the year | 1.46 | 30 | Vegetation protection limit cf. Regulation 920/2016. |
| Highest daily mean value of the year | 49 |  | Measured 30 August in easterly wind |
| Lowest daily mean value of the year | 0.0 |  |  |
| Particulate matter (PM10) |  |  |  |
| Average concentration over the year | 5.8 | 40 | Vegetation protection limit cf. Regulation 920/2016. |
| Highest daily mean value of the year | 15.9 | 50 | Vegetation protection limit cf. Regulation 920/2016. Allowed to exceed 35x over the year. Measured 15 January in south-south-westerly wind |
| Lowest daily mean value of the year | 1.7 |  |  |
| Particulate matter (PM2.5) |  |  |  |
| Average concentration over the year | 7.7 | 20 | Vegetation protection limit cf. Regulation 920/2016. |
| PAHs - Benzo(a)pyrene |  |  |  |
| Measured concentration April – October (filters) | <210-6 | 110-3 | Environmental limits cf. Regulation 410/2008. |

TABLE B.2  Results of weather and air quality measurements at Gröf II in 2025, average concentration and highest and lowest daily and hourly values ​​of the year, along with defined environmental limits that apply outside the industrial area.

| Parameters | Concentration (µg/m3) | Reference value (µg/m3) | Explanation |
| --- | --- | --- | --- |
| F (filters, Apríl - October) |  |  |  |
| Average HF concentration | 0.03 |  |  |
| Average HF concentration (April - September) | 0.03 | 0.3 | Norðurál Grundartangi’s operation permit |
| Highest measured average HF concentration | 0.88 |  | Measured 21 July in southerly wind |
| Lowest measured average HF concentration | 0.0 |  |  |
| Average total HF concentration | 0.04 |  |  |
| Average total HF concentration (April - September) | 0.04 | 0.4 | Norðurál Grundartangi’s operation permit |
| Highest measured average total HF concentration | 0.88 |  | Measured 21 July in southerly wind |
| Lowest measured average total HF concentration | 0.0 |  |  |
| F (filters, throughout the year) |  |  |  |
| Average HF concentration | 0.07 |  |  |
| Highest measured average HF concentration | 3.08 |  | Measured 14 January in west-south-westerly wind |
| Lowest measured average HF concentration | 0.0 |  |  |
| Average total HF concentration | 0.09 |  |  |
| Highest measured average total HF concentration | 3.79 |  | Measured 14 January in west-south-westerly wind |
| Lowest measured average total HF concentration | 0.0 |  |  |
| SO2 |  |  |  |
| Average annual concentration (gaseous, in filters) | 3.51 |  |  |
| Annual average (automatic measurements) | 5.22 | 20 | Vegetation protection limit cf. Regulation 920/2016. |
| Highest daily mean value of the year | 465.2 | 125 | Vegetation protection limit cf. Regulation 920/2016. Allowed to exceed 3x over the year. Measured 21 July in southerly wind |
| Lowest daily mean value of the year | 0.0 |  |  |
| Highest hourly mean value of the year | 1667.3 | 350 | Vegetation protection limit cf. Regulation 920/2016. Allowed to exceed 24x over the year. Measured 21 July in east-south-easterly wind |
| H2S |  |  |  |
| Average concentration over the year | 0.6 | 5 | Vegetation protection limit cf. Regulation 514/2010. |
| Highest daily mean value of the year | 3.9 | 50 | Vegetation protection limit cf. Regulation 514/2010. Allowed to exceed 5x over the year. Measured 6 April in northerly wind |
| Lowest daily mean value of the year | 0.0 |  |  |
| NO2 |  |  |  |
| Average concentration over the year | 1.70 | 40 | Vegetation protection limit cf. Regulation 920/2016. |
| Highest daily mean value of the year | 15.9 | 75 | Vegetation protection limit cf. Regulation 920/2016. Allowed to exceed 7x over the year. Measured 8 January in north-north-eastely wind |
| Lowest daily mean value of the year | 0.2 |  |  |
| Highest hourly mean value of the year | 57.7 | 200 | Vegetation protection limit cf. Regulation 920/2016. Measured 8 January in north-north-eastely wind |
| NOx |  |  |  |
| Average concentration over the year | 1.85 | 30 | Vegetation protection limit cf. Regulation 920/2016. |
| Highest daily mean value of the year | 18.3 |  | Measured 8 January in north-northeasterly wind |
| Lowest daily mean value of the year | 0.2 |  |  |
| Particulate matter (PM10) |  |  |  |
| Average concentration over the year | 6.50 | 40 | Vegetation protection limit cf. Regulation 920/2016. |
| Highest daily mean value of the year | 59.6 | 50 | Vegetation protection limit cf. Regulation 920/2016. Allowed to exceed 35x over the year. Measured 20 July in south-southeasterly wind |
| Lowest daily mean value of the year | 1.1 |  |  |
| Particulate matter (PM2.5) |  |  |  |
| Average concentration over the year | 3.24 | 20 | Vegetation protection limit cf. Regulation 920/2016. |

TABLE B.3  Results of air quality measurements at Melahverfi in 2025, average concentration and highest and lowest daily and hourly values ​​of the year, along with defined environmental limits that apply outside the industrial area.

| Parameters | Concentration (µg/m3) | Reference value (µg/m3) | Explanation |
| --- | --- | --- | --- |
| SO2 |  |  |  |
| Average concentration over the year | 0.4 | 20 | Vegetation protection limit cf. Regulation 920/2016. |
| Highest daily mean value of the year | 5.9 | 125 | Vegetation protection limit cf. Regulation 920/2016. Allowed to exceed 3x over the year. Measured 23 September in east-southeasterly wind. |
| Lowest daily mean value of the year | 0.0 |  |  |
| Highest hourly mean value of the year | 17.1 | 350 | Vegetation protection limit cf. Regulation 920/2016. Allowed to exceed 24x over the year. Measured 17 December in easterly wind. |
| H2S |  |  |  |
| Average concentration over the year | 0.63 | 5 | Vegetation protection limit cf. Regulation 514/2010. |
| Highest daily mean value of the year | 3.2 | 50 | Vegetation protection limit cf. Regulation 514/2010. Allowed to exceed 5x over the year. Measured 14 November in east-southeasterly wind. |
| Lowest daily mean value of the year | 0.0 |  |  |

### Fluoride in the atmosphere

Figure B.1 shows the average concentration of gaseous fluorine (HF) in the atmosphere each month at Kríuvarða and Gröf II in 2025 and for comparison in 2024. The concentration fluctuates somewhat between months, but the average concentration was higher than the previous year at Kríuvarða and at Gröf II.

Fluoride in the atmosphere by month

1.4

1.2

µg HF/m³

1.0

0.8

0.6

0.4

0.2

0.0

2024	2025

Kríuvarða

Jan.	Feb.	Mar.	Apr.	May	June	July	Aug.	Sept.	Oct.	Nov.	Dec.

1.4

1.2

µg HF/m³

1.0

0.8

0.6

0.4

0.2

0.0

2024	2025

Gröf II

Jan.	Feb.	Mar.	Apr.	May	June	July	Aug.	Sept.	Oct.	Nov.	Dec.

FIGURE B.1	The average concentration of gaseous fluoride (HF) in the atmosphere at the monitoring stations Kríuvarða and Gröf II during 2025

### Sulphur in dust and the atmosphere

Figure B.2 shows sulphur concentrations measured in dust samples collected on filters at the monitoring stations.  Sulphur in airborne dust originates from a variety of natural sources, including sea spray and windblown soil and road dust. Consequently, sulphur concentrations in dust are not considered a direct indicator of industrial pollution in Hvalfjörður. In 2025, sulphur concentrations were lower than in previous years and remained below the average recorded over the past decade.

Sulfur in dust

2.00

1.60

µg SO₂/m³

1.20

Kríuvarða	Gröf II	Hálsnes

0.80

0.40

0.00

1997	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

FIGURE B.2	Average sulphur concentration in dust samples collected on filters at Kríuvarða and Gröf II during 2025.

Figure B.3 presents the results of automatic sulphur dioxide (SO2 measurements using fluorescence-based monitoring systems. Monthly average concentrations are shown for Kríuvarða, Gröf II, and Melahverfi in 2025, together with corresponding results from 2024.  At Kríuvarða, average SO2 concentrations were generally lower in 2025 than in the previous year, although some month-to-month variability was observed. At Gröf II, concentrations were somewhat higher than in 2024 and showed considerable seasonal variation.

SO2 in the atmosphere each month

25

2024 automatic measurements	2025 automatic measurements

20

µg SO₂/m³

15

10

5

0

Kríuvarða

Jan.	Feb.	Mar.	Apr.	May	June	July	Aug.	Sept.	Oct.	Nov.	Dec.

Gröf II

25

20

µg SO₂/m³

15

10

5

0

Jan.	Feb.	Mar.	Apr.	May	June	July	Aug.	Sept.	Oct.	Nov.	Dec.

25

Melahverfi

20

µg SO₂/m³

15

10

5

0

Jan.	Feb.	Mar.	Apr.	May	June	July	Aug.	Sept.	Oct.	Nov.	Dec.

FIGURE B.3	Average monthly sulphur dioxide (SO₂) concentrations measured automatically at Kríuvarða, Gröf II, and Melahverfi in 2024 and 2025.

### Particulate matter in air

Figures B.4 and B.5 show average monthly concentrations of particulate matter, PM10 og PM2.5, measured at Kríuvarða and Gröf II. At Kríuvarða, concentrations of both PM10 and PM2,5 were higher in 2025 than in 2024 for all months except January. At Gröf II, PM10 PM₁₀ concentrations varied considerably between months and years.

Particulate matter (PM10) in the atmosphere each month.

20

2024	2025

µg PM10/m³

15

Kríuvarða

10

5

0

Jan.

Feb.

Mar.

Apr.

May

June

July

Aug.

Sep.

Oct.

Nov.

Dec.

20	Gröf II

µg PM10/m³

15

10

5

0

Jan.

Feb.

Mar.

Apr.

May

June

July

Aug.

Sep.

Oct.

Nov.

Dec.

FIGURE B.4	Average monthly concentrations of particulate matter (PM10) at Kríuvarða and Gröf II during 2024 and 2025.

Particulate matter (PM2.5) in the atmosphere each month.

20

2024	2025

µg PM2.5/m³

15

Kríuvarða

10

5

0

Jan.

Feb.

Mar.

Apr.

May

June

July

Aug.

Sep.

Oct.

Nov.

Dec.

20

Gröf II

µg PM2.5/m³

15

10

5

0

Jan.

Feb.

Mar.	Apr.

May	June

July

Aug.

Sep.

Oct.

Nov.

Dec.

FIGURE B.5	Average monthly concentrations of particulate matter (PM2,5) at Kríuvarða and Gröf II during 2024 and 2025. No measurements were received from Gröf II in 2024 due to a failure.

### Dissolved constituents in precipitation

From January to July, precipitation samples were collected at Kríuvarða and Gröf II and analyzed for sodium, chloride, calcium, and nitrate. Results are expressed as deposition loads per square meter per month at each monitoring location. Figures B.6 and B.7 show average concentrations of sodium and chloride in precipitation at Kríuvarða and Gröf II, together with comparative results from Hálsnes, where monitoring is conducted every third year. At Gröf II, sodium and chloride concentrations were broadly consistent with the average values recorded during the past decade. In contrast, measurements at Kríuvarða were substantially higher than long-term averages.

Sodium in precipitation

1.500

Kríuvarða

Gröf II

Hálsnes

mg Na+/m²

1.000

500

0

1997

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

FIGURE B.6	Average sodium concentration in precipitation at Kríuvarða and Gröf II.

Chloride in precipitation

Kríuvarða	Gröf II	Hálsnes

1.500

mg Cl-/m²

1.000

500

0

1997

2016

2017	2018

2019	2020

2021

2022

2023

2024

2025

FIGURE B.7	Average chloride concentration in precipitation at Kríuvarða and Gröf II.

# Appendix C	Freshwater

### Monitoring results

Table C.1 summarizes the average concentrations of all monitored parameters during 2025 and compares them, where applicable, with maximum concentrations specified in Icelandic Drinking Water Regulation No. 536/2001. The table also presents the minimum and maximum values recorded for each parameter during the monitoring period.

TABLE C.1	Summary of monitoring results for freshwater in 2025, including average, minimum, and maximum concentrations of all measured parameters, together with applicable drinking-water quality standards.

| Measurement location (number of samples) | Fluoride | Chloride | Sulfate | Acidity | Conductivity |
| --- | --- | --- | --- | --- | --- |
|  | (µg/L) | mg/L | mg/L | pH | (µS/cm) |
| Kalmansá river (6) | 167 Min 99 Max 220 | 31.4 Min 26.4 Max 37.6 | 8.8 Min 5.0 Max 12.3 | 7.36 Min 7.06 Max 7.70 | 210 Min 120 Max 271 |
| Urriðaá river (6) | 147 Min 105 Max 180 | 29.6 Min 25.6 Max 37.6 | 6.4 Min 4.0 Max 8.1 | 7.35 Min 7.18 Max 7.64 | 184 Min 153 Max 209 |
| Berjadalsá river (3) | 25 Min 22 Max 30 | 16.4 Min 12.1 Max 20.0 | 2.9 Min 2.1 Max 3.9 | 7.34 Min 7.15 Max 7.60 | 96 Min 83 Max 103 |
| Fossá river (2) | 33 Min 26 Max 40 | 11.7 Min 11.6 Max 11.8 | 1.9 Min 1.5 Max 2.4 | 7.54 Min 7.48 Max 7.60 | 103 Min 85 Max 120 |
| Laxá river (2) | 32 Min 30 Max 34 | 9.5 Min 8.3 Max 10.6 | 2.0 Min 1.5 Max 2.4 | 7.52 Min 7.37 Max 7.66 | 85 Min 80 Max 89 |
| Kúludalsá river (3) | 35 Min 29 Max 40 | 19.9 Min 15.7 Max 25.5 | 4.6 Min 3.3 Max 6.3 | 7.13 Min 6.65 Max 7.56 | 94 Min 83 Max 108 |
| Maximum limit cf. Reg. 536/2001 | 1,500 | 250 | 250 | 6.5 – 9.5 | N/A |

### Influence of marine inputs on water quality measurements

Chloride is routinely measured in freshwater monitoring samples as an indicator of marine influence. Throughout the monitoring period, chloride concentrations in all rivers have remained well below the maximum concentration for drinking water specified in Icelandic Regulation No. 536/2001 (250 mg Cl/L). To evaluate the influence of seawater on river chemistry, the fluoride-to-chloride (F/Cl) ratio and sulfate-to-chloride (SO₄/Cl) ratio are calculated for samples collected from Urriðaá river (Figures C.1 and C.2). In 2025, fluoride concentrations in the Urriðaá river were comparable to the average values recorded over the past decade, whereas chloride concentrations were elevated relative to the long-term average. As a result, the fluoride-to-chloride ratio was lower than in most previous years and was the lowest recorded since 2017, when fluoride concentrations were also relatively low. Fluoride concentrations showed little change between 2024 and 2025, while chloride concentrations increased noticeably.

Fluoride/chloride ratio in Urriðaá river

10

8

6

4

2

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

FIGURE C.1	Average fluoride-to-chloride ratio in the Urriðaá River, including background values from 1997 for reference.

The sulfate-to-chloride ratio was also used to assess the influence of marine inputs. As shown in Figure C.2, the sulfate-to-chloride ratio in 2025 was similar to the average recorded over the past ten years. Sulfate concentrations were considerably below the long-term average and have only been lower once since 2006. Sulfate in the river may be influenced, in part, by emissions from the industrial area at Grundartangi.

Sulfate-to-chloride ratio in Urriðaá river

0.7

0.6

0.5

0.4

0.3

0.2

0.1

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0.0

FIGURE C.2	Average sulfate-to-chloride ratio in Urriðaá river, including background values from 1997 for reference.

### Statistical analysis of freshwater monitoring data

Freshwater environmental monitoring has been conducted annually since 1997. To evaluate long-term trends and variability in water quality, statistical analyses were performed on monitoring data from Kalmansá, Urriðaá, and Berjadalsá for the period 1997–2025. For each parameter, 95% confidence intervals around the annual mean concentrations were calculated using the t-distribution. Figures C.3–C.5 present the results of the statistical analysis of pH measurements in Kalmansá, Urriðaá and Berjadalsá, for the period 1997–2025, respectively. As the number of observations increases over time, the statistical uncertainty decreases, which is reflected in narrower confidence intervals.

14

12

Acidity (pH)

10

8

6

4

2

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

FIGURE C.3	Mean pH in Kalmansá with 95% confidence intervals.

14

Acidity (pH)

12

10

8

6

4

2

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

FIGURE C.4	Mean pH in Urriðaá river with 95% confidence intervals.

14

12

Acidity (pH)

10

8

6

4

2

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

FIGURE C.5	Mean pH in Berjadalsá with 95% confidence intervals.

Figures C.6–C.11 present the statistical analyses of fluoride concentrations and fluoride-to-chloride ratios in the three monitored rivers, Kalmansá, Urriðaá and Berjadalsá.

16

12

F/Cl

8

4

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

FIGURE C.6	Mean fluoride-to-chloride ratio in Kalmansá with 95% confidence intervals.

250

200

µg F/L

150

100

50

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

FIGURE C.7	Mean fluoride concentration in Kalmansá with 95% confidence intervals.

16

12

F/Cl

8

4

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

FIGURE C.8	Mean fluoride concentration in Kalmansá with 95% confidence intervals.

250

200

µg F/L

150

100

50

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

FIGURE C.9	Mean fluoride concentration in Urriðaá with 95% confidence intervals.

16

12

F/Cl

8

4

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

FIGURE C.10	Mean fluoride-to-chloride ratio in Berjadalsá with 95% confidence intervals.

250

200

µg F /L

150

100

50

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

FIGURE C.11	Mean fluoride concentration in Berjadalsá with 95% confidence intervals.

# Appendix D	Seawater by flow pits

TABLE D.1   Average, maximum, and minimum concentrations of constituents measured in seawater samples collected in May and July 2025, together with average concentrations measured at the corresponding reference locations during 2021–2025, where available. Environmental quality standards have not been established for parameters marked with an asterisk (*). Fluoride analyses could not be reported because the samples were mishandled during laboratory processing.

| Monitoring site | Cyanide*** (Free) mg/L | Min Max | Average 2021-2025 | Cyanide*** (Total) mg/L | Min Max | Average 2021-2025 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 Eastern end - 1 m | <0.0010 | <0.0010 | 0.002 | 0.004 | <0.0010 | 0.003 |
|  |  | 0.0017 |  |  | 0.011 |  |
| 2 Eastern end - 4 m | <0.0010 | <0.0010 | 0.002 | <0.0010 | <0.0010 | 0.002 |
|  |  | <0.0010 |  |  | 0.0014 |  |
| 3 Eastern side - 1 m | <0.0010 | <0.0010 | 0.003 | 0.0015 | <0.0010 | 0.004 |
|  |  | 0.0019 |  |  | 0.0034 |  |
| 4 Eastern side - 4 m | <0.0010 | <0.0010 | 0.002 | 0.0013 | <0.0010 | 0.002 |
|  |  | <0.0010 |  |  | 0.0028 |  |
| 5 Center - 1 m | <0.0010 | <0.0010 | 0.002 | 0.0067 | <0.0010 | 0.003 |
|  |  | <0.0010 |  |  | 0.019 |  |
| 6 Center - 4 m | <0.0010 | <0.0010 | 0.002 | <0.0010 | <0.0010 | 0.003 |
|  |  | <0.0010 |  |  | 0.004 |  |
| 3 Western side - 1 m | <0.0010 | <0.0010 | 0.002 | <0.0010 | <0.0010 | 0.002 |
|  |  | <0.0010 |  |  | <0.0010 |  |
| 8 Western side - 4 m | <0.0010 | <0.0010 | 0.002 | <0.0010 | 0.0016 | 0.002 |
|  |  | <0.0010 |  |  | <0.0010 |  |
| 9 Western end - 1 m | <0.0010 | <0.0010 | 0.002 | <0.0010 | <0.0010 | 0.002 |
|  |  | <0.0010 |  |  | <0.0010 |  |
| 10 Western end - 4 m | <0.0010 | <0.0010 | 0.002 | <0.0010 | <0.0010 | 0.002 |
|  |  | <0.0010 |  |  | <0.0010 |  |
| Kalastaðir | <0.0010 | <0.0010 | 0.002 | <0.0010 | <0.0010 | 0.002 |
|  |  | <0.0010 |  |  | 0.0013 |  |
| Center of fjord | <0.0010 | <0.0010 | 0.002 | <0.0010 | 0.001 | 0.002 |
|  |  | <0.0010 |  |  | <0.0010 |  |

APPENDIX D – SEAWATER BY FLOW PITS

TABLE D.2	Average, maximum, and minimum concentrations of constituents measured in seawater samples collected in May and July 2025, together with average concentrations measured at the corresponding reference locations during 2021–2025, where available. Environmental quality standards have not been established for parameters marked with an asterisk (*).

| Monitoring site | As Min µg/L Max |  | Cr Min µg/L Max |  | Cu Min µg/L Max |  | Ni Min µg/L Max |  | Pb Min µg/L Max |  | Zn Min µg/L Max |  | Al Min µg/L Max |  | Fe* Min µg/L Max |  | P* Min µg/L Max |  | V* Min µg/L Max |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Eastern end - 1 m | 1.75 | 1.49 2.01 | 0.35 | 0.20 0.50 | <0.5 | <0.5 0.8 | <0.5 | <0.5 0.81 | <0.3 | <0.3 <0.3 | <2.0 | <2 <2 | 14 | 8 18 | 27 | 9 18 | <40 | <40 <40 | 2.31 | 2.19 2.4 |
| 2 Eastern end - 4 m | 1.62 | 1.42 1.93 | 0.51 | 0,21 0.95 | <0.5 | <0.5 0.5 | 0.96 | <0.5 2.37 | <0.3 | <0.3 <0.3 | <2,0 | <2 <2 | 11 | 8 13 | 38 | 10 16 | <40 | <40 <40 | 2.27 | 2.12 2.53 |
| 3 Eastern side - 1 m | 1.63 | 1.42 1.9 | 0.35 | 0.27 0.44 | <0.5 | <0.5 0.8 | <0.5 | <0.5 0.72 | <0.3 | <0.3 <0.3 | <2.0 | <2 <2 | 13 | 9 19 | 50 | 12 23 | <40 | <40 <40 | 2.28 | 2.16 2.46 |
| 4 Eastern side - 4 m | 1.74 | 1.48 2.21 | 0.27 | 0.13 0.36 | <0.5 | <0.5 0.6 | <0.5 | <0.5 <0.5 | <0.3 | <0.3 <0.3 | <2.0 | <2 <2 | 15 | 8 23 | 48 | 11 25 | <40 | <40 53 | 2.16 | 2.02 2.36 |
| 5 Center - 1 m | 1.72 | 1.49 2.1 | 0.57 | 0.28 1.1 | 0.65 | 0,7 1 | <0.5 | <0.5 <0.5 | <0.3 | <0.3 <0.3 | <2.0 | <2 2.64 | 23 | 20 29 | 65 | 16 31 | <40 | <40 <40 | 2.30 | 2.02 2.46 |
| 6 Center - 4 m | 1.83 | 1.39 2.27 | 0.26 | 0.22 0.30 | 0.60 | 0.5 0.7 | <0.5 | <0.5 <0.5 | <0.3 | <0.3 <0.3 | <2.0 | <2 2.28 | 15 | 6 23 | 36 | 7 29 | <40 | <40 <40 | 2.18 | 2.07 2.3 |
| 3 Western side - 1 m | 2.06 | 1.35 2.33 | 0.30 | 0.30 0.44 | 0.77 | 0.5 0.9 | <0.5 | <0.5 <0.5 | <0.3 | <0.3 <0.3 | <2.0 | <2 <2 | 23 | 9 40 | 77 | 12 42 | <40 | <40 <40 | 2.25 | 2.07 2.44 |
| 8 Western side - 4 m | 0.72 | 1.43 2.05 | 0.16 | 0.13 0.32 | 0.80 | 0.6 1 | <0.5 | <0.5 <0.5 | <0.3 | <0.3 <0.3 | <2.0 | <2 <2 | 14 | 9 22 | 38 | 13 25 | <40 | <40 <40 | 2.26 | 2.1 2.54 |
| 9 Western end - 1 m | 1.70 | 1.45 1.94 | 0.40 | 0.18 0.61 | 0.80 | 0.7 0.9 | <0.5 | <0.5 0.53 | <0.3 | <0.3 <0.3 | <2.0 | <2 <2 | 14 | 7 25 | 44 | 14 30 | <40 | <40 <40 | 2.26 | 2.09 2.46 |
| 10 Western end - 4 m | 1.98 | 1.5 2.45 | 0.27 | 0.16 0.37 | 0.75 | 0.7 0.8 | <0.5 | <0.5 0.52 | <0.3 | <0.3 <0.3 | <2.0 | <2 2.8 | 12 | 7 17 | 39 | 18 21 | <40 | <40 <40 | 2.34 | 2.16 2.49 |
| Kalastaðir | 1.96 | 1.56 2.36 | 0.28 | 0.16 0.39 | 0.65 | 0.6 0.7 | <0.5 | <0.5 0.6 | <0.3 | <0.3 <0.3 | 3.11 | 3.12 3.11 | 85 | 7 210 | 179 | 12 471 | 44 | <40 <40 | 2.84 | 2.2 3.65 |
| Center of fjord | 1.89 | 1.51 2.22 | 0.22 | 0.17 0.28 | <0.5 | <0.5 0.5 | <0.5 | <0.5 <0.5 | <0.3 | <0.3 0.3 | <2.0 | <2 <2 | 7 | 2 10 | 13 | <4 24 | <40 | <40 <40 | 2.34 | 2.21 2.48 |
| Kalastaðir Average 2021-2025 | 1.89 |  | 0.89 |  | 1.1 |  | 1.0 |  | 0.5 |  | 3.31 |  | 66 |  | 120 |  | 68 |  | 2.55 |  |
| Center of fjord Average 2021-2025 | 1.65 |  | 0.46 |  | 0.7 |  | 0.5 |  | 0.3 |  | 2.24 |  | 23 |  | 33 |  | 39 |  | 2.24 |  |

Samples collected on 28 July 2025 from monitoring stations 1, 6, 8, 9, and 10, as well as from Kalastaðir, were compromised during laboratory handling and could not be included in the assessment.

Very low or no risk of ecological impact

Low risk of ecological impact

Potential impact on sensitive ecosystems

Likely ecological impact

Consistently unsuitable water quality for ecosystem health

# Appendix E	Vegetation

Results from vegetation monitoring conducted north and south of Hvalfjörður are presented in Sections E.1 and E.2. Statistical analyses of vegetation monitoring data collected between 1997 and 2025 are provided in Section E.3. Vegetation samples are collected at different times during the growing season. June sampling reflects the period of rapid plant growth, while September sampling represents conditions near the end of the growing season. Additional grass samples were collected in July. In total, 14 monitoring locations were sampled in 2025, including sites on both sides of Hvalfjörður and reference sites in Skorradalur.

### Fluoride in grass

Figure E.1 presents fluoride concentrations in grass measured in 2007, when all smelter production lines were operational, and annually during 2016–2025. Reference measurements for 1997 are included where available. Monitoring began at Hlíð in 2009 and at Gröf II (near the farmhouse) in 2010. Sampling commenced at Ferstikla and Háls í Kjós in 2011, at Hólabrú in 2018, and at Kalastaðarkot in 2022. Reference samples were collected in Skorradalur. The estimated fluoride tolerance threshold for herbivores in forage is 30 µg F/g [10], while the estimated tolerance range for grasses is 100–200 µg F/g dry weight [9]. Results from earlier monitoring years (1998–2015) are available in previous annual environmental monitoring reports published by the Icelandic Environment and Energy Agency (available online, only in Icelandic).

Fluoride in grass

50

Stekkjarás

40

Reference values for feed

µg F/g

30

20

10

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	July	September

50

40

µg F/g

30

Reference values for feed

20

10

0

Fannahlíð

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	July	September

50

40

µg F/g

30

Reference values for feed

20

10

0

Fellsaxlarkot

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	July	September

50

Gröf II by the farmhouse

40

Reference values for feed

µg F/g

30

20

10

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	July	September

FIGURE E.1	Fluoride concentrations in grass and corresponding 1997 baseline values, where available.

Fluoride in grass (continued)

50

40

µg F/g

30

Reference values for feed

20

10

0

Hlíð

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	July	September

50

40

µg F/g

30

The estimated tolerance limit for herbivores.

20

10

0

Hólabrú

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	July	September

50

40

µg F/g

30

Reference values for feed

20

10

0

Félagsgarður

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	July	September

50

40

µg F/g

30

Reference values for feed

20

10

0

Fossbrekka

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	July	September

FIGURE E.1	(continued) Fluoride concentrations in grass and corresponding 1997 baseline values.

Fluoride in grass (continued)

50

40

µg F/g

30

Reference values for feed

20

10

0

Ferstikla

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	July	September

50

40

µg F/g

30

Reference values for feed

20

10

0

Háls in Kjós

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	July	September

50

40

µg F/g

30

The estimated tolerance limit for herbivores.

20

10

0

Kalastaðarkot

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	July	September

FIGURE E.1	(continued) Fluoride concentrations in grass and corresponding 1997 baseline values, where applicable.

Fluoride in grass (continued)

50

40

µg F/g

30

Reference values for feed

20

10

0

Reynivellir

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	July	September

50

40

µg F/g

30

Reference values for feed

20

10

0

Skorradalur

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	September	Reference value for feed

FIGURE E.1	(continued) Fluoride concentrations in grass and corresponding 1997 baseline values.

### Fluoride in leaves

Figure E.2 shows average fluoride concentrations in leaves from thirteen monitoring locations for 2007 (when all pots were in operation) and for the period 2016–2025, together with reference measurements from 1997 where applicable. Monitoring of fluoride in leaves began at Gröf II and Hlíð in 2009. Additional monitoring sites were added over time, including Ferstikla and Háls in Kjós in 2011, Hólabrú in 2018, and Kalastaðarkot in 2022. Measurements in Skorradalur were undertaken in 2013 and subsequently on an annual basis from 2016 onward. The estimated fluoride tolerance threshold for deciduous trees is 200 µg F/g dry weight [9]. Results from earlier monitoring years (1998–2015) are available in previous annual environmental monitoring reports published by the Icelandic Environment and Energy Agency (available online, only in Icelandic).

Fluoride in leaves

200

160

Estimated tolerance threshold for deciduous trees

Stekkjarás

µg F/g

120

80

40

0

1997

2007

2016

2017	2018

2019

June

2020

September

2021

2022	2023

2024

2025

200

160

Estimated tolerance threshold for deciduous trees

Fannahlíð

µg F/g

120

80

40

0

1997

2007

2016	2017

2018

2019	2020

2021

2022

2023

2024

2025

June	September

200

160

Estimated tolerance threshold for deciduous trees

Fellsaxlarkot

µg F/g

120

80

40

0

1997	2007

2016

2017

2018	2019

2020

2021

2022

2023

2024

2025

June	September

FIGURE E.2	Average fluoride concentrations in leaves during spring and autumn, together with 1997 baseline values where available.

200

160

Estimated tolerance threshold for deciduous trees

Ferstikla

µg F/g

120

80

40

0

1997	2007	2016	2017	2018	2019	2020	2021

June	September

2022	2023	2024	2025

200

160

Estimated tolerance threshold for deciduous trees

Gröf II by the farmhouse

µg F/g

120

80

40

0

1997	2007	2016

2017	2018

2019

June

2020

September

2021

2022	2023	2024	2025

200

160

Estimated tolerance threshold for deciduous trees

Hólabrú

µg F/g

120

80

40

0

1997	2007	2016	2017	2018

2019

June

2020

September

2021

2022	2023

2024	2025

200

160

Estimated tolerance threshold for deciduous trees

Hlíð

µg F/g

120

80

40

0

1997	2007

2016	2017

2018	2019	2020	2021

June	September

2022

2023	2024	2025

FIGURE E.2	(continued) Average fluoride concentrations in leaves during spring and autumn, together with 1997 baseline values for reference.

200

160

Estimated tolerance threshold for deciduous trees

Félagsgarður

µg F/g

120

80

40

0

1997

2007

2016	2017

2018	2019	2020	2021	2022	2023

June	September

2024	2025

200

160

Estimated tolerance threshold for deciduous trees

Fossbrekka

µg F/g

120

80

40

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	September

200

160

Estimated tolerance threshold for deciduous trees

Háls in Kjós

µg F/g

120

80

40

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

June	September

200

160

Estimated tolerance threshold for deciduous trees

Reynivellir

µg F/g

120

80

40

0

1997	2007

2016	2017

2018	2019	2020	2021	2022	2023

June	September

2024

2025

FIGURE E.2	(continued) Average fluoride concentrations in leaves during spring and autumn, together with 1997 baseline values for reference.

200

160

Estimated tolerance threshold for deciduous trees

Kalastaðarkot

µg F/g

120

80

40

0

1997	2007	2016	2017	2018	2019	2020	2021	2022

June	September

2023

2024	2025

200

160

Estimated tolerance threshold for deciduous trees

Skorradalur

µg F/g

120

80

40

0

1997	2007	2016	2017	2018	2019	2020	2021

June	September

2022	2023	2024	2025

FIGURE E.2	(continued) Average fluoride concentrations in leaves during spring and autumn, together with 1997 baseline values for reference.

### Statistical Analysis of Vegetation Monitoring Data

Vegetation monitoring has been conducted annually since 1997. To evaluate long-term variability and trends in fluoride concentrations, statistical analyses were performed using data collected during 1997–2025. For each parameter, 95% confidence intervals around the annual mean concentrations were calculated using the t-distribution. These analyses provide an indication of both temporal trends and the uncertainty associated with the measurements. Figures E.3–E.6 summarize the statistical analyses of fluoride concentrations in different vegetation types.

Figure E.3 presents average fluoride concentrations in grass north and south of Hvalfjörður, while Figure E.4 presents corresponding results for deciduous tree leaves.  Figures E.5 and E.6 show fluoride concentrations in one-year-old and two-year-old conifer needles, respectively.

Fluoride in grass

35

The estimated tolerance limit for herbivores.

30

25

µg F/g

20

15

10

5

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

North of Hvalfjörður	South of Hvalfjörður

FIGURE E.3	Average fluoride concentrations in grass north and south of Hvalfjörður with 95% confidence intervals.

Fluoride in leaves

200

Estimated tolerance threshold for deciduous trees

160

µg F/g

120

80

40

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

North of Hvalfjörður	South of Hvalfjörður

FIGURE E.4	Average fluoride concentrations in deciduous tree leaves north and south of Hvalfjörður with 95% confidence intervals.

100

Estimated tolerance limit of coniferous trees (30 - 100 µg/g)

    - year conifer needles

80

µg F/g

60

40

20

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

North of Hvalfjörður	South of Hvalfjörður

FIGURE E.5	Average fluoride concentrations in one-year-old conifer needles north and south of Hvalfjörður with 95% confidence intervals.

100

Estimated tolerance limits for conifers (30–100 µg F/g) are shown for comparison.

    - year conifer needles

80

µg F/g

60

40

20

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

North of Hvalfjörður	South of Hvalfjörður

FIGURE E.6	Average fluoride concentrations in two-year-old conifer needles north and south of Hvalfjörður with 95% confidence intervals.

# Appendix F	Herbivores (sheep and horses)

Fluoride concentrations were measured in bone ash from the jawbones of 95 lambs and adult sheep. In addition, the heads and teeth were examined by a veterinarian for signs of fluorosis and other dental abnormalities. Assessments included tooth wear, enamel condition, discoloration, tooth morphology, tooth loss, loose or broken teeth, jaw osteomyelitis, gum recession, and tartar accumulation. Table F.1 provides an overview of the monitoring farms that supplied sheep heads for analysis and inspection, while Table F.2 summarizes fluoride concentrations in jawbone ash. All fluoride concentrations are reported on a dry-weight basis [14]. Sheep head samples collected for fluoride analysis and veterinary examination.

TABLE F.1    Overview of monitoring farms.

|  | Received samples | Number of samples |  | Condition of the enamel on the front teeth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Normal |  | With changes |  |
|  |  | Lambs | Adult sheep | Lambs | Adult sheep | Lambs | Adult sheep |
| North of Hvalfjörður | Eystra Miðfell | 0 | 3 |  | 3 |  |  |
|  | Eystri Leirárgarðar | 4 | 4 | 4 | 32) |  |  |
|  | Hóll | 4 | 4 | 4 | 4 |  |  |
|  | Hrafnabjörg | 4 | 4 | 3 | 4 | 1 |  |
|  | Innri Hólmur | 3 | 1 | 3 | 1 |  |  |
|  | Skipanes | 4 | 4 | 3 | 4 | 1 |  |
|  | Skorholt | 4 | 4 | 3 | 32) |  |  |
|  | Vestri Reynir | 2 | 0 | 2 |  |  |  |
|  | Vogatunga | 4 | 4 | 4 | 3 |  | 1 |
| South of Hvalfjörður | Kiðafell | 4 | 2 | 4 | 2 |  |  |
|  | Miðdalur1) | 4 | 4 | 4 | 4 |  |  |
|  | Fell1) | 4 | 4 | 4 | 4 |  |  |
| Reference samples – sheep |  |  |  |  |  |  |  |
|  | Bjarnarhöfn (Snæfellsnes) | 4 | 4 |  |  |  |  |
|  | Skjaldfönn (N-Ísafjörður) | 4 | 4 |  |  |  |  |
| Total |  | 49 | 46 |  |  |  |  |
| 1) A new farm 2) All front teeth were missing in one sheep |  |  |  |  |  |  |  |

TABLE F.2  Summary of fluoride concentrations in the jawbone ash of lambs and adult sheep, including mean, minimum, and maximum values recorded at each monitoring farm. Based on Norwegian studies, fluoride concentrations exceeding 900 μg F/g dry weight are considered indicative of a potential risk of dental fluorosis in young deer [13,17]. Values above this threshold are highlighted in yellow. Concentrations exceeding 2,000 μg F/g dry weight, which have been associated with dental fluorosis in deer, are highlighted in red.

| Farm | Lambs | Adult sheep |  |
| --- | --- | --- | --- |
|  | Fluoride (µg F/g dry weight) | Fluoride (µg F/g dry weight) | Age |
| North of Hvalfjörður |  |  |  |
| Eystra Miðfell | - | 892 | 3 |
|  | - | 1,377 | 4 |
|  | - | 1,740 | 5 |
|  | - | - |  |
| Average concentration / (Min-Max) | No sample- | 1,336 / (892-1,740) |  |
| Eystri Leirárgarðar | 95 | 590 | 8 |
|  | 81 | 687 | 8 |
|  | 104 | 493 | 7 |
|  | 75 | 639 | 8 |
| Average concentration / (Min-Max) | 89 / (75-104) | 602 / (493-639) |  |
| Hóll | 126 | 695 | 7 |
|  | 104 | 695 | 4 |
|  | 107 | 681 | 7 |
|  | 73 | 548 | 6 |
| Average concentration / (Min-Max) | 103 / (73-126) | 655 / (548-695) |  |
| Hrafnabjörg | 197 | 894 | 5 |
|  | 136 | 808 | 5 |
|  | 101 | 955 | 6 |
|  | 61 | 786 | 5 |
| Average concentration / (Min-Max) | 124 / (61-197) | 860/ (786-955) |  |
| Innri Hólmur | 268 | 1,186 | 5 |
|  | 278 | - |  |
|  | 279 | - |  |
|  | - | - |  |
| Average concentration / (Min-Max) | 275 / (268-279) | 1,186 |  |
| Skipanes | 45 | 1.149 | 6 |
|  | 64 | 986 | 6 |
|  | 68 | 960 | 4 |
|  | 70 | 1,205 | 6 |
| Average concentration / (Min-Max) | 125 / (70-216) | 1,075 / (960 – 1,205) |  |
| Skorholt | 135 | 774 | 6 |
|  | 163 | 1,061 | 5 |
|  | 149 | 833 | 7 |
|  | 110 | 1,114 | 7 |
| Average concentration / (Min-Max) | 139 / (110-163) | 946 / (774-1.114) |  |
| Vestri Reynir | 164 | - |  |
|  | 200 | - |  |
|  | - | - |  |
|  | - | - |  |
| Average concentration / (Min-Max) | 182 / (164-200) | No sample |  |
| Vogatunga | 94 | 1,296 | 8 |
|  | 117 | 805 | 9 |
|  | 122 | 1,076 | 7 |
|  | 116 | 984 | 8 |
| Average concentration / (Min-Max) | 112 / (94-122) | 1,040 / (805-1,296) |  |

| Farm | Lambs | Adult sheep |  |
| --- | --- | --- | --- |
|  | Fluoride (µg F/g dry weight) | Fluoride (µg F/g dry weight) | Age |
| South of Hvalfjörður |  |  |  |
| Kiðafell | 76 | 644 | 7 |
|  | 70 | 530 | 8 |
|  | 141 | - |  |
|  | 69 | - |  |
| Average concentration / (Min-Max) | 89 / (69-141) | 587 / (530-644) |  |
| Miðdalur | 125 | 719 | 4 |
|  | 118 | 785 | 8 |
|  | 117 | 884 | 4 |
|  | 157 | 921 | 5 |
| Average concentration / (Min-Max) | 129 / (117-157) | 827 / (719-921) |  |
| Fell | 202 | 965 | 6 |
|  | 176 | 1,011 | 6 |
|  | 148 | 808 | 6 |
|  | 162 | 760 | 6 |
| Average concentration / (Min-Max) | 172 / (148-202) | 886 / (760-1,011) |  |
| Reference sites |  |  |  |
| Bjarnarhöfn | 48 | 94 | 4 |
|  | 59 | 117 | 5 |
|  | 72 | 122 | 7 |
|  | 61 | 116 | 5 |
| Average concentration / (Min-Max) | 60 / (48-72) | 112 / (94-122) |  |
| Skjaldfönn | 97 | 209 | 8 |
|  | 147 | 314 | 9 |
|  | 89 | 209 | 8 |
|  | 74 | 293 | 7 |
| Average concentration / (Min-Max) | 102 / (94-147) | 256 / (209-314) |  |

### Fluoride in lambs and adult sheep by area

Fluoride in the jawbone of lambs by area

Figure E.1 presents fluoride concentrations in grass measured in 2007, when all smelter production lines were operational, and annually during 2016–2025. Reference measurements for 1997 are included where available. Results from earlier monitoring years (1998–2015) are available in previous annual environmental monitoring reports published by the Icelandic Environment and Energy Agency (available online, only in Icelandic).

This area is considered the least affected by fluoride emissions because of prevailing wind patterns. Monitoring in recent years has been limited to a single farm in this area, see Figure F.1. Although no baseline data are available from 1997 for this area, the average fluoride concentration measured in 2025 was significantly lower than the corresponding values recorded in both 1999 and 2007 (Figure F.11).

2000

Lambs - East of Grundartangi, north of Hvalfjörður

Associated with dental fluorosis in deer

1500

µg F/g

1000

Considered capable of causing dental fluorosis in deer

500

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

Hrafnabjörg

FIGURE F.1	Average fluoride concentrations in the jawbones of lambs from farms located east of the industrial area.

Figure F.2 presents fluoride concentrations measured in the jawbones of lambs from monitoring farms located southwest of Akrafjall. Owing to the prevailing wind direction, this area is expected to experience the greatest influence from fluoride emissions originating from the aluminium smelter. The average fluoride concentration measured in lamb jawbones from this area in 2025 was significantly higher than the corresponding concentration measured in 1997. However, concentrations were significantly lower than those recorded in 2007, when all smelter potlines were in operation (Figure F.12).

2000

Lambs - southwest of Akrafjall

Associated with dental fluorosis in deer

1500

µg F/g

1000

Considered capable of causing dental fluorosis in deer

500

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

Innri-Hólmur	Vestri Reynir

FIGURE F.2	Average fluoride concentrations in lamb jawbones from farms southwest and west of the industrial area, including reference values from 1997

Figure F.3 presents fluoride concentrations in lamb jawbones from farms located north and northwest of the industrial area. No suitable lamb samples were obtained from Eystra Miðfell in 2025 because collection was unsuccessful at the slaughterhouse. The average fluoride concentration measured in 2025 did not differ significantly from levels recorded in either 1997 or 2007 (Figure F.13).

2.000

Lambs - North and northwest of Grundartangi

Associated with dental fluorosis in deer

Considered capable of causing dental fluorosis in deer

1.500

µg F/g

1.000

500

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

Vogatunga	Eystri Leirárgarðar	Skipanes	Skorholt	Hóll	Eystra Miðfell

FIGURE F.3	Average fluoride concentration in the jawbones of lambs from farms north and northwest of the industrial area, together with reference values from 1997.

Figure F.4 shows fluoride concentrations in lamb jawbones from farms south of Hvalfjörður. Miðdalur was included as a new monitoring farm in this area, while Fell was reintroduced after last being sampled in 2007. The average fluoride concentration in the jawbones of lambs from this area in 2025 was significantly higher than in 1997 but significantly lower than in 2007 (Figure F.14).

2000

Associated with dental fluorosis in deer

Lambs - South of Hvalfjörður:

1500

µg F/g

1000

Considered capable of causing dental fluorosis in deer

500

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

Fell	Kiðafell	Miðdalur	Meðalfell	Eyjar

FIGURE F.2	Average fluoride concentration in the jawbones of lambs from farms south of Hvalfjörður, including reference values from 1997.

Fluoride in adult sheep by region

Monitoring east of the industrial area is conducted at Hrafnabjörg (Figure F.5). In 2025, the average fluoride concentration measured in adult sheep from this farm was not significantly different from levels observed in either 1997 or 2007 (Figure F.17).

3.000

Adult sheep - East of Grundartangi, north of Hvalfjörður

µg F/g

2.000

Associated with dental fluorosis in deer

Considered capable of causing dental fluorosis in deer

1.000

0

1997	2007	2016	2017	2018	2020	2021	2022	2023	2024	2025

Hrafnabjörg

FIGURE F.5	Average fluoride concentration in the jawbones of adult sheep from farms east of the industrial area, together with reference values from 1997. No samples were received in 2019.

Figure F.6 presents average fluoride concentrations in the jawbones of adult sheep from monitoring farms southwest of the industrial area. No adult sheep samples were received from Vestri Reynir in 2025, and only one sample was obtained from Innri Hólmur. The average fluoride concentration in this region did not differ significantly from that measured in 2007 (Figure F.18).

3.000

Adult sheep - southwest of Akrafjall

Associated with dental fluorosis in deer

µg F/g

2.000

1.000

Considered capable of causing dental fluorosis in deer

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

Innri Hólmur	Vestri Reynir

FIGURE F.6	Average fluoride concentration in the jawbones of adult sheep from farms southwest and west of the industrial area.

Figure F.7 shows average fluoride concentrations in adult sheep from farms north and northwest of the industrial area. No significant differences were detected between the 2025 results for average fluoride in the jawbones of adult sheep and those recorded in either 1997 or 2007 (Figure F.19).

3.000

Adult sheep - North and northwest of Grundartangi

µg F/g

2.000

Associated with dental fluorosis in deer

Considered capable of causing dental fluorosis in deer

1.000

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

Vogatunga	Eystri Leirárgarðar	Skipanes	Skorholt	Hóll	Eystra Miðfell

FIGURE F.7	Average fluoride concentration in the jawbones of adult sheep from farms north and northwest of the industrial area, including reference values from 1997.

Figure F.8 presents average fluoride concentrations in adult sheep from farms south of Hvalfjörður, an area also influenced by emissions from the aluminium smelter. Fell and Miðdalur were included as new monitoring sites in 2025, while no samples were obtained from Meðalfell. The average fluoride concentration in adult sheep from this region was significantly higher in 2025 than in both 1997 and 2007 (Figure F.20).

3.000

Adult sheep - south of Hvalfjörður

µg F/g

2.000

Associated with dental fluorosis in deer

Considered capable of causing dental fluorosis in deer

1.000

0

1997	2007	2016	2017	2018	2019	2020	2021	2022	2023	2024	2025

Fell	Kiðafell	Miðdalur	Meðalfell

FIGURE F.8	Average fluoride concentration in the jawbones of adult sheep from farms south of Hvalfjörður, including reference values from 1997.

APPENDIX F – HERBIVORES (SHEEP AND HORSES)

### Examination of teeth and joints in live grazing animals (sheep and horses)

Tables F.3 and F.4 summarize the veterinary examination of teeth and forelimb joints in live sheep and horses.

TABLE F.3	Summary of dental and forelimb joint examinations in sheep. Dental assessments were conducted using the scoring system developed in 1974 [22] and described in Table 8.2. in Section 8.

| Farm | Date | n | Age | Front teeth on the bottom right side |  |  |  | Front teeth on the bottom left side |  |  |  | Average | Molars |  | Joints |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ilr4 | ilr3 | ilr2 | ilr1 | ill1 | ill2 | ill3 | ill4 |  | mr | ml |  |
| Fell | 27 Jan. 2026 | 9 | 3.6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Miðdalur | 31 Jan. 2026 | 10 | 3.7 | 0 | 0 | 0 | 0.11 | 0.10 | 0.10 | 0.10 | 0 | 0.05 | 0 | 0 | 0 |
| Hrafnabjörg | 27 Jan. 2026 | 10 | 4.4 | 0 | 0 | 0 | 0 | 0.10 | 0 | 0 | 0 | 0.01 | 0 | 0 | 0 |
| Vogatunga | 27 Jan. 2026 | 10 | 4.3 | 0 | 0 | 0 | 0.13 | 0 | 0 | 0 | 0.10 | 0.03 | 0.10 | 0.10 | 0 |
| Eystra Miðfell | 27 Jan 2026 | 10 | 4.4 | 0.17 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.02 | 0 | 0 | 0.10 |
| Kiðafell | 31 Jan 2026 | 10 | 4.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Abbreviations: n = number; iur = incisor upper right; iul = incisor upper left; mr = molars right; ml = molars left

TABLE F.4	Summary of dental and forelimb joint examinations in horses. Dental assessments were conducted using the scoring system developed in 1974 [22] and described in Table 8.2. in Section 8.

| Farm | Date | n | Age | Front teeth upper right |  |  | Front teeth upper left |  |  | Front teeth bottom right |  |  | Front teeth bottom left |  |  | Average | Joints |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | iur3 | iur2 | iur1 | iul1 | iul2 | iul3 | ilr3 | ilr2 | ilr1 | ill1 | ill2 | ill3 |  |  |
| Ytri-Hólmur | 31 Jan 2026 | 6 | 14 | 0 | 0 | 0.67 | 0.33 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.08 | 0 |
| Litla Fellsöxl | 27 Jan 2026 | 6 | 22 | 0.17 | 0.67 | 0.67 | 0.67 | 0.73 | 0.17 | 0 | 0 | 0 | 0 | 0 | 0 | 0.26 | 0 |
| Miðdalur | 31 Jan 2026 | 6 | 13 | 0 | 0 | 0.83 | 0.50 | 0.33 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.14 | 0 |
| Kalastaðakot | 31 Jan 2026 | 5 | 14 | 0 | 0 | 0.40 | 0.20 | 0.20 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.07 | 0 |
| Meðalfell | 14.02.2026 | 6 | 15 | 0 | 0 | 0.67 | 0.67 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.11 | 0 |

Abbreviations: n = number; iur = incisor upper right; iul = incisor upper left; ilr: incisor lower right; ill = incisor lower left.

### Statistical Analysis of Fluoride in Sheep

Fluoride concentrations in sheep jawbone ash have been monitored annually since 1997. To assess long-term trends and variability in the data, statistical analyses were performed using monitoring results collected between 1997 and 2025.  Mean concentrations and associated 95% confidence intervals were calculated using a t-distribution.

Results of the statistical analyses are presented in Figures F.9–F.14 for lambs and Figures F.15–F.20 for adult sheep. For lambs, average fluoride concentrations in 2025 remained higher than those measured at the start of monitoring in 1997 but lower than concentrations recorded in 2007, when all smelter furnaces were operational. No meaningful change was observed between 2024 and 2025.

Lambs

Fluoride in lambs - all monitoring farms

2000

Associated with dental fluorosis in deer

1500

µg F/g

1000

Considered capable of causing dental fluorosis in deer

500

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

Average of all monitoring points

FIGURE F.9	Mean fluoride concentration in lamb jawbones from all monitoring farms with 95% confidence intervals.

1.500

µg F/g

1.000

Considered capable of causing dental

fluorosis in deer

500

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

North of Hvalfjörður	South of Hvalfjörður

FIGURE F.10	Comparison of mean fluoride concentrations in lamb jawbones from farms north and south of Hvalfjörður, with 95% confidence intervals.

Lambs - East of Grundartangi, north of Hvalfjörður

2000

Associated with dental fluorosis in deer

150

1500

Annual F emissions (t)

100

µg F/g

1000

Considered capable of causing dental fluorosis in deer

.

50

500

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0	0

Fluoride measured in the jawbones of lambs	Fluoride emissions

FIGURE F.11	Mean fluoride concentration in lambs from the area east of Grundartangi, shown with 95% confidence intervals and total annual fluoride emissions from the smelter.

Considered capable of causing dental fluorosis in deer

1500

100

µg F/g

Annual F emissions (t)

1000

50

500

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0	0

Fluoride measured in the jawbones of lambs	Fluoride emissions

FIGURE F.12	Mean fluoride concentration in lambs from the area southwest of Akrafjall, shown with 95% confidence intervals and total annual fluoride emissions from the smelter.

2000

Lambs - north and northwest of Grundartangi

Associated with dental fluorosis in deer

150

Considered capable of causing dental fluorosis in deer

1500

100

µg F/g

Annual F emissions (t)

1000

50

500

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0	0

Fluoride measured in the jawbones of lambs	Fluoride emissions

FIGURE F.13	Mean fluoride concentration in lambs from farms north and northwest of Grundartangi, shown with 95% confidence intervals and total annual fluoride emissions from the smelter.

2000

Associated with dental fluorosis in deer

150

Considered capable of causing dental fluorosis in deer

1500

100

µg F/g

Annual F emissions (t)

1000

50

500

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0	0

Fluoride measured in the jawbones of lambs	Fluoride emissions

FIGURE F.14	Mean fluoride concentration in lambs from farms south of Hvalfjörður, shown with 95% confidence intervals and total annual fluoride emissions from the smelter.

Adult sheep

Average fluoride concentrations in adult sheep were higher in 2025 than at the start of monitoring in 1997, but similar to levels measured in 2007, when all the smelter’s furnaces were in operation. No substantial change was observed between 2024 and 2025.

2000

Fluoride in adult sheep - all monitoring farms

Associated with dental fluorosis in deer

Considered risk of dental fluorosis in deer

1500

µg F/g

1000

500

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

Average of all monitoring points

FIGURE F.15	Mean fluoride concentration in adult sheep from all monitoring farms, with 95% confidence intervals.

2.000

Adult sheep - comparison between north and south of Hvalfjörður.

Associated with dental fluorosis in deer

Considered capable of causing dental fluorosis in deer

1.500

µg F/g

1.000

500

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

North of Hvalfjörður	South of Hvalfjörður

FIGURE F.16	Comparison of fluoride concentrations in jawbone ash from adult sheep north and south of Hvalfjörður.

3500	150

Associated with dental fluorosis in deer

Considered capable of causing dental fluorosis in deer

3000

2500

100

µg F/g

Annual F emissions (t)

2000

1500

50

1000

500

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0	0

Fluoride measured in the jawbones of adult sheep	Fluoride emissions

FIGURE F.17	Mean fluoride concentration in adult sheep east of Grundartangi, shown with 95% confidence intervals and total annual fluoride emissions from the smelter.

3.500

Adult sheep - southwest of Akrafjall

150

Associated with dental fluorosis in deer

Considered capable of causing dental fluorosis in deer

3.000

2.500

µg F/g

2.000

100

Annual F emissions (t)

1.500

50

1.000

500

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0	0

Fluoride measured in the jawbones of lambs	Fluoride emissions

FIGURE F.18	Mean fluoride concentration in adult sheep southwest of Akrafjall, shown with 95% confidence intervals and total annual fluoride emissions from the smelter.

Adult sheep - North and northwest of Grundartangi

3500	150

Associated with dental fluorosis in deer

Considered capable of causing dental fluorosis in deer

3000

2500

µg F/g

2000

100

Annual F emissions (t)

1500

50

1000

500

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0	0

Fluoride measured in the jawbones of adult sheep	Fluoride emissions

FIGURE F.19	Mean fluoride concentration in adult sheep from farms north and northwest of Grundartangi, shown with 95% confidence intervals and total annual fluoride emissions from the smelter.

3500

Adult sheep - south of Hvalfjörður

150

Associated with dental fluorosis in deer

Considered capable of causing dental fluorosis in deer

3000

2500

µg F/g

2000

100

Annual F emissions (t)

1500

50

1000

500

1997

1998

1999

2000

2001

2002

2003

2004

2005

2006

2007

2008

2009

2010

2011

2012

2013

2014

2015

2016

2017

2018

2019

2020

2021

2022

2023

2024

2025

0

FIGURE F.20	Mean fluoride concentration in adult sheep from farms south of Hvalfjörður, shown with 95% confidence intervals and total annual fluoride emissions from the smelter.